# ADR-STREAM-008 — Implicit Stream Access for Stream-Capable Computation

Status: LOCKED — SUPERSEDES CONFLICTING PORTIONS OF ADR-STREAM-007
Architecture baseline: Aurora v1.2r
Date: August 3, 2026

# Canonical Rule

Stream access is implicit when a stream-capable arithmetic, packed, multiply, MAC/MAS, or reduction instruction selects Q0 or Q1 as an operand or destination. Standalone POP, PUSH, and PEEK remain explicit stream-transfer instructions. MOV and CMOV are pure GPR operations and never consume, produce, peek, or otherwise access stream state.

# Locked Decisions

Q0 and Q1 are architectural stream operands available only to instruction classes whose definitions explicitly mark them stream-capable.

For a stream-capable instruction, naming Qn as a source implicitly consumes exactly one logical stream transfer from Qn.

For a stream-capable instruction, naming Qn as a destination implicitly produces exactly one logical stream transfer to Qn.

Stream consume/produce is part of the arithmetic or reduction instruction itself; no preceding POP or following PUSH is required.

Standalone POP, PUSH, and PEEK remain available for cases that need an explicit transfer without a simultaneous arithmetic or reduction operation.

MOV is GPR-to-GPR only. MOV cannot name Q0/Q1 and cannot access stream metadata.

CMOV is GPR-to-GPR only. CMOV cannot name Q0/Q1, and its true and false paths have identical stream, memory, MMIO, address, count, ordinal, and exception side effects.

Packed lane-wise CMOV consumes only existing predicate state and GPR values; it never conditionally advances a stream.

Arithmetic and DSP formats are biased toward stream sources and accumulator destinations. Encoding priority is consume/reduce first, GPR-result forms second, and stream-producing forms where useful and affordable.

One-source reductions may select one GPR or one stream source and write only A0 or A1.

Two-source reductions such as DOT, SAD, and comparison-count may select supported GPR/stream source combinations; paired Q0/Q1 consumption receives highest stream-form priority.

The A0/A1 accumulator is an explicit destination selector and an implicit source only when the reduction's ACC bit is set. INIT replaces/initializes the selected accumulator.

A stream-capable instruction that names two stream sources consumes both atomically: either both transfers and the arithmetic result retire, or neither stream advances.

A stream-capable instruction with a stream destination produces only after successful completion. On fault, no stream source, stream destination, GPR, accumulator, predicate, address, count, or ordinal state is partially updated.

PEEK never advances stream state. It remains the only standalone non-consuming stream-data transfer.

ADDR_NEXT, ADDR_LAST, ORD_NEXT, and ORD_LAST remain system registers accessed through system-register transfer instructions.

POPMETA Rd:Rd+1, Qn, meta remains reserved, not allocated. If later adopted, it uses an aligned register-pair destination and atomically returns popped data plus selected metadata.

The statement in ADR-STREAM-007 that all stream transfer is only through POP/PUSH/PEEK is withdrawn and must not be repeated.

R13 remains an optional link register. Compiler call analysis may allocate R13 as a GPR in leaf functions or whenever the return address is saved or dead.

# Examples

ADD R4, Q0, R5            ; implicitly consume Q0, write R4
ADD Q1, Q0, R5            ; implicitly consume Q0 and produce Q1
REDUCE.ADD A0, Q0, ACC    ; implicitly consume Q0 and accumulate into A0
DOT A1, Q0, Q1, ACC       ; atomically consume Q0 and Q1
POP R4, Q0                ; explicit standalone consume
PEEK R5, Q0               ; explicit non-consuming read
PUSH Q1, R6               ; explicit standalone produce

MOV R4, Q0                ; illegal
CMOV.GT R4, Q0            ; illegal
CMOV.GT Q1, R4            ; illegal

# Rationale

Aurora is stream-first. Folding stream transfers into computation removes redundant data-movement instructions and lets short hardware loops perform consume/compute/reduce efficiently. Keeping MOV and CMOV pure prevents data-dependent stream alignment and preserves deterministic side effects.