# ADR-DSP-012 — COMPLEX16 Result Width and Overflow Handling

Status: LOCKED
Architecture baseline: Aurora v1.2j
Date: August 2, 2026

# Decision

Aurora retains a uniform 32-bit general-purpose register width. No 33-bit GPR format or special extra result bit is added.

COMPLEX16 MUL accepts packed signed sources [imaginary:16 | real:16].

The exact mathematical real and imaginary components may each require 33 signed bits.

COMPLEX16 MUL writes one aligned 64-bit destination pair: Rd receives the low 32 bits of the real component and Rd+1 receives the low 32 bits of the imaginary component.

If either exact component is outside the signed 32-bit range, the corresponding sticky DSP overflow status is set.

The architecture provides separate sticky real and imaginary overflow indications, COVR and COVI, plus an aggregate COV condition defined as COVR OR COVI.

Overflow does not trap or interrupt by default.

An implementation may provide a privileged trap-enable control that converts newly detected complex overflow into a precise DSP-overflow exception, but this is optional and disabled after reset.

COMPLEX16 MAC and MAS accumulate the exact widened real and imaginary components into A0 and A1 respectively, preserving the full product range subject to the existing accumulator-width and overflow rules.

COMPLEX16 MUL therefore uses compact packed results, while MAC/MAS provide the exact accumulation path for precision-sensitive DSP kernels.

The prior v1.2i four-register exact-result layout is superseded.

# Result Layout

Sources:
  Rs = [ Ai:16 | Ar:16 ]
  Rt = [ Bi:16 | Br:16 ]

Exact internal values:
  Real33 = Ar*Br - Ai*Bi
  Imag33 = Ar*Bi + Ai*Br

Architectural MUL result:
  Rd   = Real33[31:0]
  Rd+1 = Imag33[31:0]
  COVR = sticky(COVR OR !fits_signed32(Real33))
  COVI = sticky(COVI OR !fits_signed32(Imag33))

# Rationale

Changing the register file, forwarding network, scoreboard, ABI, debugger state, and context format for a single extra complex-result bit would be disproportionate. Sticky overflow reporting preserves visibility of lost range, while A0/A1 retain an exact accumulation path for the workloads where that precision matters.