# ADR-DSP-015 — COMPLEX8 Packed Multiply Mode

Status: LOCKED
Architecture baseline: Aurora v1.2m
Date: August 2, 2026

# Decision

Aurora restores COMPLEX8 as a first-class packed complex multiply mode.

COMPLEX8 is selected through the existing sticky MUL_COMPLEX interpretation state together with 8-bit packed element width.

Each 32-bit source contains two signed complex values packed as [imag1:8 | real1:8 | imag0:8 | real0:8].

For each complex lane k, the exact internal components are Real_k = Ar_k*Br_k - Ai_k*Bi_k and Imag_k = Ar_k*Bi_k + Ai_k*Br_k.

Each exact COMPLEX8 component may require 17 signed bits.

Narrow COMPLEX8 MUL writes one aligned 64-bit destination pair containing [imag1:16 | real1:16 | imag0:16 | real0:16].

Each 17-bit exact component is reduced to its low 16 bits; sticky COVR and COVI indicate real or imaginary overflow in any packed lane.

The implementation may retain a non-architectural per-lane overflow mask for debug, trace, or optional profile use.

CONJ_A and CONJ_B modifiers apply independently to both packed complex lanes.

COMPLEX8 MAC and MAS perform horizontal two-lane complex reduction into A0 and A1: A0 accumulates Real_0 + Real_1 and A1 accumulates Imag_0 + Imag_1.

The exact widened products and sums are injected into the existing shared compressor/reduction network before accumulation.

Independent per-lane complex accumulators are not added in the baseline architecture.

COMPLEX8 uses the same overflow, trap-disable-by-default, and sticky status philosophy as COMPLEX16.

COMPLEX32 remains deferred.

# Packed Layout

Source register:
  [31:24] imag1
  [23:16] real1
  [15:8]  imag0
  [7:0]   real0

Destination pair:
  [63:48] imag1_result16
  [47:32] real1_result16
  [31:16] imag0_result16
  [15:0]  real0_result16

# Rationale

Aurora already contains packed 8-bit multiplication, horizontal reduction, sign routing, sticky complex overflow, and COMPLEX16 cross-product control. COMPLEX8 therefore regularizes the mode system at modest incremental cost and directly benefits low-precision communications, SDR, correlation, beamforming, and packed FFT kernels.