# Aurora Architecture Decision Log

Current consolidated preservation copy • August 3, 2026

# Purpose and Authority

This preservation copy combines the original ADR register, standalone later ADRs, and late-session decisions that were not yet assigned permanent ADR numbers. Newer records supersede conflicting older wording.

# Original ADR Register

| ADR ID | Decision | Status | Primary document | Notes |
| --- | --- | --- | --- | --- |
| ADR-0001 | Mission: MCU + low/mid DSP consolidation | Frozen | AUR-ARCH-001 | Core product identity |
| ADR-0002 | 16-bit base ISA | Frozen | AUR-ARCH-001 | Extension words for larger encodings |
| ADR-0003 | 16 unified 32-bit GPRs | Frozen | AUR-ARCH-001 | Unified register file |
| ADR-0004 | Paired-register 64-bit emulation | Frozen | AUR-ARCH-001 | No native 64-bit core |
| ADR-0005 | R13 LR, R14 SP, R15 PC | Frozen | AUR-ARCH-003 | Conditional LR allocation |
| ADR-0006 | 40-bit accumulators A0/A1 | Frozen | AUR-ARCH-001 | DSP accumulators |
| ADR-0007 | Q0/Q1 stream engines | Frozen | AUR-ARCH-001 | Overlay on R0/R4 |
| ADR-0008 | Packed modes + L8/L16 | Frozen | AUR-ARCH-001 | Lane-preserving modes |
| ADR-0009 | Four-stage pipeline | Frozen | AUR-ARCH-007 | In-order |
| ADR-0010 | Four-entry pairing window | Frozen | AUR-ARCH-007 | Compiler-assisted |
| ADR-0011 | Symmetric dual-issue lanes | Frozen | AUR-ARCH-007 | Shared execution resources |
| ADR-0012 | No renaming / no ROB | Frozen | AUR-ARCH-007 | Deterministic execution |
| ADR-0013 | Backward-taken predictor | Frozen | AUR-ARCH-007 | Simple branch prediction |
| ADR-0014 | Optional ITCM/DTCM | Frozen | AUR-ARCH-006 | Profile-scalable memory |
| ADR-0015 | Single countdown timer | Frozen | AUR-ARCH-006 | Software multiplexing |
| ADR-0016 | Compiler-first scheduling | Frozen | AUR-TOOL-001 | Hardware repairs locally |
| ADR-0017 | Machine-readable ISA as source of truth | Frozen | AUR-ARCH-004 | Generates tools/docs |

# Standalone Later ADR Records

| ID | Record title | Current status | Repository file | Supersession / notes |
| --- | --- | --- | --- | --- |
| ADR-2DSTREAM-001 | ADR-2DSTREAM-001 — Sticky MODE Instruction and Two-Dimensional Stream Stepping | Locked/Accepted | architecture-records/ADR_2DSTREAM_001_MODE_and_2D_Stream_Stepping.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-SYS-002 | ADR-SYS-002 — Privilege, Wait/Halt, and Context Transfer Architecture | LOCKED | architecture-records/ADR-SYS-002_Privilege_Wait_Halt_and_Context_Transfer.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-ISA-019 | ADR-ISA-019 — Extension Instructions and Stream/Stack Bounds Faults | LOCKED | architecture-records/ADR-ISA-019_Extension_and_Bounds_Faults.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-ISA-020 | ADR-ISA-020 — NOP-Terminated Hardware Loop | LOCKED | architecture-records/ADR-ISA-020_NOP_Terminated_Hardware_Loop.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-STREAM-006 | ADR-STREAM-006 — Pinned Circular Stream Mode | LOCKED | architecture-records/ADR-STREAM-006_Pinned_Circular_Stream_Mode.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-DSP-011 | ADR-DSP-011 — Complex Multiply Mode and Widened Result Layout | LOCKED | architecture-records/ADR-DSP-011_Complex_Multiply_Mode_and_Widened_Layout.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-DSP-012 | ADR-DSP-012 — COMPLEX16 Result Width and Overflow Handling | LOCKED | architecture-records/ADR-DSP-012_COMPLEX16_Result_Width_and_Overflow.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-DSP-013 | ADR-DSP-013 — Multiply Width and Interpretation Mode Bits | LOCKED | architecture-records/ADR-DSP-013_Multiply_Width_and_Interpretation_Mode_Bits.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-DSP-014 | ADR-DSP-014 — Rounding and Saturation Mode Bits | LOCKED | architecture-records/ADR-DSP-014_Rounding_and_Saturation_Mode_Bits.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-DSP-015 | ADR-DSP-015 — COMPLEX8 Packed Multiply Mode | LOCKED | architecture-records/ADR-DSP-015_COMPLEX8_Packed_Multiply_Mode.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-DSP-016 | ADR-DSP-016 — Unary Complex Conjugate Instruction | LOCKED | architecture-records/ADR-DSP-016_Unary_Complex_Conjugate_Instruction.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-MEM-009 | ADR-MEM-009 — Required Near-Memory DMA and Reduction/Stream Separation | LOCKED | architecture-records/ADR-MEM-009_Required_Near_Memory_DMA_and_Separation.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-ISA-021 | ADR-ISA-021 — Contextual ZR, Unified Compare, and Packed Predicate Flags | LOCKED | architecture-records/ADR-ISA-021_Contextual_ZR_Unified_Compare_and_Predicates.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-STREAM-007 | ADR-STREAM-007 — Explicit Stream Access and Side-Effect-Free Moves | Partially superseded | architecture-records/ADR-STREAM-007_Explicit_Stream_Access_and_Pure_Moves.docx | Pure MOV/CMOV rule retained; stream-access exclusivity superseded by ADR-STREAM-008. |
| ADR-STREAM-008 | ADR-STREAM-008 — Implicit Stream Access for Stream-Capable Computation | LOCKED — SUPERSEDES CONFLICTING PORTIONS OF ADR-STREAM-007 | architecture-records/ADR-STREAM-008_Implicit_Stream_Access_for_Computation.docx | Standalone record preserved in decision-logs/architecture-records. |
| ADR-FE-006 | ADR-FE-006 — Two-Entry Prefetch Buffer and Eight-Word Hardware Loop | LOCKED | architecture-records/ADR-FE-006_Two_Entry_Prefetch_and_Eight_Word_Loop.docx | Standalone record preserved in decision-logs/architecture-records. |

# Late-Session Decisions Pending Formal ADR Numbering

| Provisional ID | Decision | Status | Source | Notes |
| --- | --- | --- | --- | --- |
| LATE-STREAM-POPMETA | POPMETA aligned-pair stream consume with value + ORD/ADDR metadata | Locked | RECENT_DECISIONS_SNAPSHOT.md | Register-pair destination; atomic data+metadata; exact final encoding still to be reconciled with opcode table. |
| LATE-DSP-TRACK | Fused TRACKMIN/TRACKMAX compare-and-track pair operation | Reserved / benchmark | RECENT_DECISIONS_SNAPSHOT.md | Not baseline; compare against POPMETA + CMP + two CMOVs. |
| LATE-FE-REPLAY | Resident replay for short backward-relative branches in the eight-word front-end window | Locked | RECENT_DECISIONS_SNAPSHOT.md | Target derived from relative offset; no refetch on hit. |
| LATE-FE-SINGLEENTRY | Compiler-enforced single-entry rule for resident-replay regions | Locked | RECENT_DECISIONS_SNAPSHOT.md | Hardware does not detect or enforce interior entry. |
| LATE-FE-LOOPROLE | Explicit hardware loops retained alongside resident replay | Locked | RECENT_DECISIONS_SNAPSHOT.md | Hardware loop removes decrement/test/branch and final-exit misprediction. |
| LATE-SCHED-001 | Priority-first EDF scheduler with optional latest-safe-start ordering | Design locked | scheduler/README.md | Higher priority first; within priority EDF or finish-deadline minus estimated remaining execution. |
| LATE-SCHED-002 | Dependency urgency inheritance and mandatory latest-safe-start preemption | Design locked | scheduler/README.md | Inherit highest blocked priority, then earliest urgency at that priority. |
| LATE-RED-COUNT | Comparison-count reduction | Not locked | RECENT_DECISIONS_SNAPSHOT.md | Explicitly retained only as a candidate. |

Governance note: provisional entries are preserved as decisions but should receive permanent ADR identifiers in the next reconciliation pass.