# ADR-DSP-011 — Complex Multiply Mode and Widened Result Layout

Status: LOCKED
Architecture baseline: Aurora v1.2i
Date: August 2, 2026

# Locked Decisions

All Aurora multiply operations produce mathematically widened results; complex mode does not introduce truncating multiply semantics.

Complex multiplication is selected through sticky arithmetic MODE state and does not allocate separate CMUL or CMAC opcodes.

The baseline complex mode is COMPLEX16, operating on one signed complex value packed into each 32-bit source register.

For COMPLEX16, bits [15:0] contain the real component and bits [31:16] contain the imaginary component.

Given A = Ar + jAi and B = Br + jBi, multiply-family operations compute Real = Ar*Br - Ai*Bi and Imag = Ar*Bi + Ai*Br.

Each component is an exact signed 33-bit mathematical result because it combines two signed 16x16 products.

The real result is sign-extended into one aligned 64-bit destination register pair; the imaginary result is sign-extended into the next aligned 64-bit destination pair.

Complex widened outputs are not word-interleaved. Each real or imaginary component remains contiguous in its own normal aligned register pair.

Ordinary packed 2x16 multiplication remains lane-wise: each signed 16x16 product widens independently to 32 bits, and the two 32-bit lane products occupy one aligned 64-bit destination pair.

Complex MAC and MAS use A0 as the real accumulator and A1 as the imaginary accumulator.

A0 and A1 receive the signed cross-products through the existing multiplier/compressor network with mode-controlled routing and signs.

COMPLEX16 mode may include CONJ_A and CONJ_B modifier bits. Conjugation negates the selected source's imaginary component before product routing.

Complex mode affects MUL, MAC, MAS, and complex-capable DOT/reduction forms only. ADD, SUB, logic, compare, load/store, and ordinary packed operations retain their normal semantics.

Complex 32-bit multiplication is not part of the v1.2i baseline because each exact component can require 65 signed bits, exceeding one 64-bit register pair. It remains a future profile decision.

The scoreboard treats the four-register widened complex destination as two aligned register-pair writes and reserves both pairs until completion.

# Result Layout

Sources:
  Rs = [ Ai:16 | Ar:16 ]
  Rt = [ Bi:16 | Br:16 ]

Destination beginning at aligned even register Rd:
  Rd:Rd+1     = sign_extend_64(Ar*Br - Ai*Bi)
  Rd+2:Rd+3   = sign_extend_64(Ar*Bi + Ai*Br)

# Rationale

This layout conforms to Aurora's existing packed-multiply rule: every output lane or component widens independently and is stored contiguously in ordinary aligned register-pair containers. Avoiding interleaving preserves carry, shift, scoreboard, forwarding, compiler, and debugger simplicity.