# ADR-DSP-014 — Rounding and Saturation Mode Bits

Status: LOCKED
Architecture baseline: Aurora v1.2l
Date: August 2, 2026

# Locked Decisions

Aurora adds two independent sticky DSP mode bits: ROUND and SAT.

ROUND applies only to explicit DSP operations that discard low-order bits: narrowing shifts, pack/narrow operations, and accumulator-to-register extraction.

SAT applies only to explicit conversions into a smaller destination width: pack/narrow operations and accumulator-to-register extraction.

Ordinary ADD, SUB, LSL, LSR, ASR, ROR, MOV, and other scalar operations remain mode-independent and retain exact modulo or bit-shift semantics.

When ROUND and SAT are both enabled, rounding is performed first and saturation is applied to the rounded value.

The baseline rounding rule is signed round-to-nearest-even using guard, round/sticky, and retained-LSB information.

Unsigned narrowing uses the corresponding unsigned round-to-nearest-even rule.

Saturation clamps to the signed or unsigned destination range selected by the narrowing/packing operation.

At least one saturation event sets the existing sticky DSP saturation status bit SAT_OCCURRED; packed implementations may retain a non-architectural lane mask for debug or trace.

Rounding alone does not set SAT_OCCURRED. Implementations may expose a separate inexact/rounded-away status only in an optional profile.

ROUND and SAT do not generate interrupts by default. A privileged optional DSP trap-enable may convert saturation into a precise exception, but it is disabled after reset.

An implementation may require one additional execution cycle when ROUND or SAT is active. This latency is architecturally permitted and must be reported in the implementation timing profile.

The added cycle, when present, is scoreboarded; dependent consumers wait normally and no software-visible partial result occurs.

The fast ordinary scalar ALU and basic shifter critical paths are not required to contain rounding or saturation logic.

MODECLR clears ROUND and SAT along with the other sticky arithmetic mode bits.

# Affected Operation Classes

| Operation class | ROUND | SAT | Notes |
| --- | --- | --- | --- |
| Ordinary scalar ALU and shifts | No | No | Always exact modulo/bit semantics |
| DSP narrowing right shift | Optional | No | Discards low-order bits but retains destination width |
| NARROW / PACK8 / PACK16 | Optional | Optional | Round first, then clamp |
| Accumulator extraction / ACCMOV | Optional | Optional | Wide accumulator to smaller GPR/packed destination |

# Rationale

Rounding and saturation are most valuable at precision-reduction boundaries. Limiting the mode bits to explicit narrowing, packing, and accumulator extraction avoids burdening Aurora's fast scalar ALUs while allowing fused fixed-point conversions. Permitting one extra cycle keeps timing closure and area under implementation control.