# ADR-ISA-020 — NOP-Terminated Hardware Loop

Status: LOCKED
Architecture baseline: Aurora v1.2g
Date: August 2, 2026

# Decision

Aurora provides one active zero-overhead hardware loop using LOOP #count.

LOOP uses opcode key 0xC0. The low eight instruction bits encode count-minus-one, supporting 1 through 256 iterations.

The first ordinary NOP encountered while a hardware loop is active terminates the loop body; no separate LOOPEND or NOP.LEND instruction exists.

NOP uses opcode key 0xC1. Outside an active hardware loop it has no architectural effect.

The loop body begins with the instruction immediately following LOOP and excludes the terminating NOP.

The loop body contains one through four 16-bit instructions, matching the baseline small loop-buffer capacity.

Only one hardware loop may be active. LOOP inside an active hardware loop is illegal; hardware nesting is not supported.

Branches, calls, returns, IRET, ASC/system-service entry, WFI, WFE, HALT, CTXSAVE, CTXRESTORE, fences, and other serializing or control-transfer instructions are prohibited in the body.

Ordinary ALU, shift, packed, multiply/reduction, DIVSTEP, load/store, and stream instructions are permitted subject to normal hazards and resource availability.

The first pass fetches and decodes the body into the loop buffer. Later iterations replay the buffered operations without instruction-memory access, branch prediction, or a software counter instruction.

At retirement of the terminating NOP, the hidden remaining count is decremented. If iterations remain, execution restarts at the first buffered operation; otherwise execution continues after NOP.

Count denotes total body executions. LOOP #1 executes the body once and does not replay it.

The loop is precisely interruptible between retired instructions. Interrupt entry snapshots active-loop state; IRET restores it.

The CONTROL context group used by CTXSAVE/CTXRESTORE includes loop-active, remaining-count, body length, replay position, and buffered-loop identity/state required for exact resumption.

A synchronous exception in the body preserves precise state. The faulting instruction may be restarted, and the remaining loop count is not decremented until the terminating NOP retires.

If no terminating NOP is decoded within four body instructions, or a prohibited instruction is encountered, a precise LOOP_FORMAT exception occurs before the offending instruction issues.

# Rationale

A NOP terminator avoids encoding body length in LOOP and allows one compact instruction to describe bodies of different sizes. Reusing the ordinary NOP avoids a special loop-end mnemonic while preserving a clear hardware marker. The four-instruction limit matches Aurora's small-window philosophy and targets DIVSTEP, CORDIC, CRC, short MAC, and iterative fixed-point kernels.

# Superseded Encoding

The earlier provisional LOOPSET/LOOPEND pair is superseded. Opcode 0xC0 is renamed LOOP and opcode 0xC1 is renamed NOP. No separate LOOPEND instruction remains.