# ADR-STREAM-006 — Pinned Circular Stream Mode

Status: LOCKED
Architecture baseline: Aurora v1.2h
Date: August 2, 2026

# Decision

Q0 and Q1 support a pinned circular read mode for small resident patterns.

Pinned mode retains a complete circular working set in the existing 128-bit stream staging buffer and replays it locally without further memory reads.

The pinned working set may contain 1 through 4 elements at 32-bit width, or any 8-bit/16-bit element count whose total storage does not exceed 128 bits.

Pinned mode is read-only in the baseline architecture. Pinned write streams are illegal.

Pinned mode requires circular mode. Enabling PIN while circular mode is disabled raises a precise QCFG fault.

The default phase advances only after a successful consuming stream read or pop.

Pipeline stalls, scoreboard delays, faults, interrupts, debug halt, and unsuccessful accesses do not advance the pinned phase.

QPEEK observes the current element without advancing it.

QSTEP Qn explicitly advances the current pinned circular phase by one element without transferring data.

QSTEP uses opcode key 0xB8 and is legal only for a valid pinned circular stream. Otherwise it raises a precise STREAM_STATE fault.

Aurora does not provide raw per-CPU-clock pinned-stream rotation in the baseline architecture.

Timer-, peripheral-, sample-clock-, or DMA-event-driven phase advancement is reserved as an optional SoC integration feature and is not architectural in the baseline core.

QCFG gains a PIN field. The assembler alias QSETPIN Qn, #ON/#OFF encodes as QCFG.

Entering pinned mode fills the complete working set before the first architectural consume completes; the implementation may prefill speculatively, but no partial pinned set is visible.

Reconfiguring address, width, circular extent, count, stride, or PIN clears pinned validity and requires refill.

Explicit stream invalidate, context restore, reset, or a relevant coherence invalidation clears pinned validity.

Full CTXSAVE/CTXRESTORE preserves pinned configuration, phase, validity, and resident staging contents when STREAM_CONFIG and STREAM_STAGING are selected.

Pinned mode does not suppress alignment, access-permission, or physical-memory faults during initial fill.

# Rationale

Pinned mode turns the existing stream staging buffer into a deterministic tiny resident circular store for repeated coefficients and short patterns. Advancing on successful consumption preserves precise in-order behavior through stalls and interrupts. Explicit QSTEP covers deliberate phase changes without coupling semantics to the raw CPU clock.

# Rejected Alternative

Automatic advancement every CPU clock was rejected because stalls, clock-frequency changes, debug stops, and interrupts would make the visible stream phase depend on microarchitectural timing rather than architectural consumption.