# ADR-STREAM-007 — Explicit Stream Access and Side-Effect-Free Moves

Status: LOCKED
Architecture baseline: Aurora v1.2q
Date: August 3, 2026

# Locked Decisions

Q0 and Q1 are stream-engine entities and are not ordinary GPR source or destination operands.

All stream data transfer is explicit through POP, PUSH, and PEEK instructions.

POP Rd, Qn consumes exactly one stream element and advances stream address, count, ordinal, and staging state only after successful completion.

PEEK Rd, Qn reads the current staged element without consuming it and without changing address, count, ordinal, phase, or staging pointers.

PUSH Qn, Rs produces exactly one stream element and advances stream state only after successful completion.

MOV is strictly a GPR-to-GPR operation. MOV may not read from or write to Q0/Q1 and may not access stream system state.

CMOV is strictly a GPR-to-GPR conditional copy. Its only conditional architectural effect is whether its GPR destination is replaced.

CMOV may not POP, PUSH, PEEK, access memory, access MMIO, read or write stream system registers, update stream addresses, or modify stream counts or ordinals.

The true and false paths of CMOV therefore have identical memory, stream, address-generation, and exception side effects.

In packed mode, CMOV remains lane-wise under the current predicate flags, but it still operates only on GPR lanes already read into the execution pipeline.

Conditional execution must never determine whether a stream advances. Stream progression is controlled only by explicit POP/PUSH operations that retire successfully.

Stream configuration and stream position metadata are architectural system-register state, accessed through the existing low-operand-count system-register transfer instructions.

Each stream exposes ADDR_NEXT, ADDR_LAST, ORD_NEXT, and ORD_LAST system registers.

ORD_NEXT and ORD_LAST are zero-based linear logical-element ordinals. They advance only on successful consuming or producing operations as defined by stream direction.

For packed stream transfers, ORD_LAST identifies the first scalar lane in the transferred packed group; a scalar lane ordinal is ORD_LAST plus its lane number.

Stalls, PEEK operations, faults, flushed instructions, interrupts before retirement, and debug halt do not advance stream address or ordinal state.

A combined POP-with-metadata instruction is not part of the v1.2q baseline. It remains a future encoding candidate if benchmarks show that a separate POP plus system-register read prevents important kernels from fitting the four-instruction hardware-loop buffer.

R13 remains an optional link register rather than a permanently reserved register. The compiler may allocate R13 as a GPR in leaf functions or whenever the return address is saved or dead.

# Canonical Stream Operations

| Operation | Data movement | Advances stream? | Permitted side effects |
| --- | --- | --- | --- |
| POP Rd, Qn | Stream to GPR | Yes, after successful retirement | Consume, address/count/ordinal update |
| PEEK Rd, Qn | Stream to GPR | No | GPR write only |
| PUSH Qn, Rs | GPR to stream | Yes, after successful retirement | Produce, address/count/ordinal update |
| MOV Rd, Rs | GPR to GPR | No | GPR write only |
| CMOV.cond Rd, Rs | Conditional GPR to GPR | No | Conditional GPR write only |

# Rationale

Explicit stream operations make every address-changing action visible in assembly and deterministic under stalls, predication, faults, interrupts, and debugging. Keeping MOV and CMOV pure prevents a condition result from changing which stream elements later instructions observe. Metadata remains system state rather than being disguised as GPR operands.