# ADR-SYS-002 — Privilege, Wait/Halt, and Context Transfer Architecture

Status: LOCKED
Architecture baseline: Aurora v1.2e
Date: August 2, 2026

# Decision

Aurora has exactly two architectural privilege modes: User and Supervisor.

The current privilege level is represented by one PSR privilege bit.

Reset, interrupt entry, exception entry, and ASC/system-service entry enter Supervisor mode.

IRET remains the canonical interrupt/exception return instruction and atomically restores the saved PC, PSR, previous privilege mode, and previous interrupt-enable state.

WFI (wait for interrupt), WFE (wait for event), and HALT are architectural instructions.

WFI waits until an enabled interrupt becomes serviceable. It may be used from User mode unless disabled by a supervisor control bit.

WFE waits for an implementation-defined architectural event, including interrupt, DMA/peripheral event, or explicit event signal. It may be used from User mode unless disabled by a supervisor control bit.

HALT is privileged and stops instruction execution until reset, debug resume, or an implementation-defined wake condition permitted by system control.

Aurora adds privileged CTXSAVE and CTXRESTORE instructions.

CTXSAVE/CTXRESTORE launch a small context sequencer that reuses the stream-style transfer path and the local DTCM/SRAM interface without consuming or overwriting architectural Q0/Q1 state.

The context transfer supports masks for volatile GPRs, full GPR state, PC/PSR, A0/A1, Q0/Q1 configuration, and optionally Q0/Q1 staging contents.

Context transfer begins at a precise retirement boundary, blocks normal issue, and is non-interruptible until its atomic completion boundary.

Maskable interrupts that arrive during a context transfer remain pending and are serviced afterward; they are not lost.

Reset and a true NMI/reset-class watchdog may abort the sequence according to implementation policy.

CTXRESTORE restores PSR and resume PC last, so partially restored state is never architecturally visible.

Aurora does not adopt four x86-style privilege rings and does not require paging or page faults in the baseline architecture.

# Rationale

Two privilege levels cover the realistic embedded RTOS split between kernel/handler code and application code. The wait instructions improve power efficiency and event-driven firmware. A thin context sequencer provides nearly the bandwidth of a dedicated context engine while preserving Aurora's shared-hardware philosophy and avoiding the self-clobbering problem that would occur if Q0/Q1 were used directly to save their own state.

# Context Transfer Timing Model

The transfer engine targets up to two 32-bit architectural words per cycle when the register-file ports and local memory banks permit. A basic 16-word GPR/PC/PSR context therefore has an approximate lower bound of eight transfer cycles plus launch and completion overhead. Exact timing is implementation-defined but bounded and reported by the implementation profile.

# Rejected Alternatives

Four privilege rings: rejected as complexity without useful embedded software demand.

Using Q0/Q1 directly for every context switch: rejected because an interrupted task may already own live stream state.

A wholly separate high-bandwidth context engine: rejected as unnecessary duplication.

Automatically turning R0-R4 into a stack window: rejected because it complicates the ABI, forwarding, and scoreboarding.