# DDR-0055 — Rounding and Saturation Documentation

Status: LOCKED
Date: August 2, 2026

AUR-ARCH-005 shall document ROUND and SAT as sticky MODE bits and list the exact instruction classes they affect.

The manual shall state that ordinary scalar shifts do not obey ROUND and ordinary arithmetic does not obey SAT.

All affected instruction entries shall specify operation ordering: shift/narrow, round-to-nearest-even, then saturation clamp.

Timing tables shall allow an implementation-defined additional cycle when ROUND or SAT is active.

AUR-ARCH-006 shall include Q15/Q31 audio, packed sample conversion, and accumulator-extraction examples.

The DSP status register documentation shall define SAT_OCCURRED and its explicit clear mechanism.

AUR-ARCH-005 v1.2l supersedes v1.2k as the canonical working Instruction Reference.