# DDR-0059 — ZR, Compare Aliases, and Packed Predicate Documentation

Status: LOCKED
Date: August 3, 2026

The Programmer's Model and ABI shall retain R15=PC, R14=SP, R13=LR, and R12=optional FP/GPR.

The Instruction Reference shall define ZR as register code 0xF only in operand positions where PC access is prohibited.

The Instruction Reference shall mark CMP and TST as aliases rather than independent datapath operations.

Packed-mode chapters shall document per-lane predicate flags generated by ordinary flag-producing ALU operations whose destination is ZR.

Conditional-move documentation shall describe scalar whole-register behavior and packed lane-wise behavior under the same mnemonic and opcode.

No PCMP, PSEL, PMOV, or other packed-only compare/select opcode shall be listed in the baseline ISA.

The assembler shall reject ZR in pair operands, load destinations, and any format where code 0xF retains PC semantics.

The simulator, RTL, decoder tables, scoreboard rules, and verification plan shall include contextual decoding of register code 0xF.

AUR-ARCH-005 v1.2p supersedes v1.2o as the canonical working Instruction Reference.