# DDR-0060 — Explicit Stream Access Documentation

Status: LOCKED
Date: August 3, 2026

AUR-ARCH-005 shall define POP, PUSH, and PEEK as the only baseline stream-data access instructions.

All examples that treat Q0/Q1 as ordinary MOV or CMOV operands shall be removed or corrected.

MOV and CMOV instruction entries shall explicitly prohibit stream, memory, MMIO, and system-register side effects.

The stream chapter shall document ADDR_NEXT, ADDR_LAST, ORD_NEXT, and ORD_LAST as system registers accessed through system-register transfer instructions.

The simulator and RTL shall advance stream state only when an explicit POP or PUSH successfully retires.

The compiler backend shall not fold stream consumption into MOV, CMOV, or predicated operations.

The ABI documentation shall describe R13 as an optional LR whose liveness is determined by call analysis.

AUR-ARCH-005 v1.2q supersedes v1.2p as the canonical working Instruction Reference.