# DDR-0062 — Prefetch and Hardware-Loop Window Documentation

Status: LOCKED
Date: August 3, 2026

AUR-ARCH-005 shall define the prefetch buffer as two 32-bit entries, separate from the four-word decoded loop replay buffer.

The hardware-loop chapter shall define an eight-word captured window and distinguish words from architectural instructions.

All loop examples and assembler diagnostics shall count extension words against the eight-word capacity.

The compiler and assembler shall reject captured loops that end on an incomplete instruction boundary.

The scheduler shall model the first four loop words as decoded replay and words five through eight as pinned raw words requiring normal decode.

The RTL, simulator, verification plan, and power model shall cover pairing across the storage boundary and prohibit pairing across the iteration wrap.

AUR-ARCH-005 v1.2s supersedes v1.2r as the canonical working Instruction Reference.