# Aurora Architecture v1.2 - Documentation Refresh

Positioning
Aurora is a unified embedded processor architecture designed to bridge the gap between traditional microcontrollers and real-time DSPs. It emphasizes deterministic execution, low implementation cost, compact 16-bit instructions, native 32-bit execution, and efficient streaming data processing.

## Canonical Architecture

Native 32-bit architecture with compact 16-bit ISA.

64-bit support is paired-register emulation; extra capability comes from additional cycles rather than substantially more hardware.

Four-stage pipeline with scoreboard support for multi-cycle memory and DSP operations.

Two radix-4 multiplier/compressor clusters plus a dedicated reduction tree.

Two 40-bit accumulators: A0 (R8) and A1 (R10).

Stream engines Q0 (R0/R1) and Q1 (R4/R5) with atomic 64-bit transfers.

One-cycle stream enable mask plus per-instruction local/consume override.

QPEEK, QPUSH, QPOP, QDUP, and QREPL queue operations.

Power-of-two aligned circular addressing with signed stride.

SIMD modes for packed 8-bit, packed 16-bit, and paired 64-bit emulation.

Paired 64-bit shifts/rotates reuse existing 32-bit hardware and are permitted to take multiple cycles.

Reduction operations support overwrite and fold (accumulate) semantics for ADD, DOT, SAD, AND, OR, XOR, MIN, MAX, ABSMAX, and COUNT.

## Commercial Position

Aurora targets deterministic embedded systems that combine control and signal processing: motor control, robotics, industrial automation, sensor fusion, communications, audio, imaging, and edge-AI preprocessing. The design goal is to replace the common MCU+DSP pairing with one efficient architecture.

## Design Philosophy

Spend silicon where it benefits common real-time workloads.

Prefer compiler assistance over excessive hardware complexity.

Favor additional cycles over duplicated wide datapaths for infrequent 64-bit operations.

Maintain deterministic timing and bounded multi-cycle execution.