# AUR-ARCH-005
Aurora Instruction Reference Manual

Showcase Edition v1.2 (Consolidated)
Canonical working instruction reference

# Table of Contents

1. Introduction

2. Programmer's Model

3. Instruction Encoding

4. Instruction Space Overview

5. Pipeline Overview

6. Instruction Families

7. Representative Instruction Entries

Appendix A - Register Summary

Appendix B - Opcode Summary

Appendix C - Glossary

# 1. Introduction

Aurora is a DSP-first embedded processor with deterministic in-order execution, 16-bit instructions, native 32-bit data paths, dual issue, stream engines, shared arithmetic hardware, and a compact, compiler-friendly ISA.

# 2. Programmer's Model

16 × 32-bit general-purpose registers (R0-R15)

A0/A1 accumulators

Q0/Q1 programmable stream engines with 2D addressing support

Program Status Register (PSR)

Little-endian memory model

# 3. Instruction Encoding

Aurora uses a fixed 16-bit base instruction with optional extension words for expanded immediates and encodings.

# 4. Instruction Space Overview

## 4.1 Primary Opcode Map

| 0x0 Arithmetic | 0x1 Logic | 0x2 Shift | 0x3 Multiply/ Reduction |
| --- | --- | --- | --- |
| 0x4 Load | 0x5 Store | 0x6 Branch | 0x7 Compare |
| 0x8 Packed DSP | 0x9 SIMD | 0xA Streams | 0xB System |
| 0xC Control | 0xD Mode | 0xE Extensions | 0xF Reserved |

## 4.2 Opcode Space Allocation

| Family | Relative Allocation |
| --- | --- |
| Arithmetic | Largest |
| Load/Store | Large |
| Packed DSP | Large |
| Logic | Medium |
| Branch | Medium |
| Streams | Medium |
| System/Mode | Small |
| Reserved | Future Expansion |

## 4.3 Decoder Organization

Instruction Word
        │
 Primary Opcode
   ├── Arithmetic
   ├── Logic
   ├── Memory
   ├── Streams
   └── System
        │
    Sub-opcode
        │
 ADD  SUB  CMP ...

## 4.4 Instruction Family Index

| Family | Opcode | Execution Unit | Chapter |
| --- | --- | --- | --- |
| Arithmetic | 0x0 | ALU | 6 |
| Logic | 0x1 | ALU | 6 |
| Shift | 0x2 | Shifter | 6 |
| Multiply | 0x3 | Shared Arithmetic | 6 |
| Load/Store | 0x4-0x5 | LSU | 6 |
| Branch | 0x6 | Branch Unit | 6 |
| Streams | 0xA | Stream Engine | 6 |
| System | 0xB-0xD | Control | 6 |

# 5. Pipeline Overview

IF → ID/RR → EX/MEM → WB/RET
Dual issue • Scoreboarding • Forwarding • In-order retirement

# 6. Instruction Families

## Arithmetic

Family overview, philosophy, representative instructions, pairing guidance, and design notes.

## Logic

Family overview, philosophy, representative instructions, pairing guidance, and design notes.

## Shift/Rotate

Family overview, philosophy, representative instructions, pairing guidance, and design notes.

## Load/Store

Family overview, philosophy, representative instructions, pairing guidance, and design notes.

## Branch

Family overview, philosophy, representative instructions, pairing guidance, and design notes.

## Packed DSP

Family overview, philosophy, representative instructions, pairing guidance, and design notes.

## Streams

Family overview, philosophy, representative instructions, pairing guidance, and design notes.

## System

Family overview, philosophy, representative instructions, pairing guidance, and design notes.

# 7. Representative Instruction Entry

## ADD

### Purpose

Integer addition.

### Syntax

ADD Rd,Rs,Rt
ADD Rd,Rs,#imm

### Operation

Rd ← Rs + operand

### Flags

Z N C V updated.

### Pipeline

EX/MEM execution, WB/RET retirement.

### Pairing

Pairs with most independent non-conflicting operations.

### Design Insight

Simple orthogonal arithmetic improves compiler scheduling.

# Appendices

Register Summary

Opcode Summary

Glossary

Future revisions will expand every instruction into this canonical format.