# AUR-ARCH-005 — Aurora Instruction Reference Manual

Showcase Edition v1.1

This revision incorporates the new Instruction Space Overview chapter near the front of the manual.

# Chapter 1 — Introduction

Aurora is a DSP-first embedded processor architecture with deterministic in-order execution, a compact 16-bit instruction encoding, native 32-bit data paths, and stream-oriented memory support.

# Chapter 2 — Programmer's Model

16 × 32-bit general-purpose registers

Program Status Register (PSR)

A0/A1 accumulators

Q0/Q1 programmable stream engines

# Chapter 3 — Instruction Encoding

Base instruction formats, extension words, register fields, immediates, and encoding rules.

# Chapter 4 — Instruction Space Overview (NEW)

## 4.1 Primary Opcode Map

| 0x0 Arithmetic | 0x1 Logic | 0x2 Shift | 0x3 Mul/Reduce |
| --- | --- | --- | --- |
| 0x4 Load | 0x5 Store | 0x6 Branch | 0x7 Compare |
| 0x8 Packed DSP | 0x9 SIMD | 0xA Streams | 0xB System |
| 0xC Control | 0xD Mode | 0xE Extension | 0xF Reserved |

## 4.2 Opcode Space Allocation

The ISA dedicates the largest portion of opcode space to arithmetic, memory access, and DSP-oriented instructions. Reserved regions are intentionally retained for future architectural expansion.

| Family | Relative Space |
| --- | --- |
| Arithmetic | Largest |
| Load/Store | Large |
| Packed DSP | Large |
| Logic | Medium |
| Branch | Medium |
| Streams | Medium |
| System/Mode | Small |
| Reserved | Remaining |

## 4.3 Decoder Organization

Instruction Word
      │
Primary Opcode
 ┌────┼────┐
Arithmetic  Logic  Memory
      │
  Sub-opcode
      │
 ADD  SUB  CMP ...

## 4.4 Instruction Family Index

| Family | Opcode | Execution Unit | Chapter |
| --- | --- | --- | --- |
| Arithmetic | 0x0 | Integer ALU | 5 |
| Logic | 0x1 | Integer ALU | 6 |
| Shift/Rotate | 0x2 | Shifter | 7 |
| Multiply/Reduction | 0x3 | Shared Arithmetic | 8 |
| Load/Store | 0x4-0x5 | LSU | 9 |
| Branch | 0x6 | Branch Unit | 10 |
| Streams | 0xA | Stream Engine | 11 |
| System | 0xB-0xD | Control | 12 |

# Remaining Chapters

The remainder of the manual follows with instruction-family chapters, appendices, opcode tables, timing information, and examples.