# Instruction Space Overview (Insert for AUR-ARCH-005)

These pages are intended to appear near the front of the Aurora Instruction Reference, immediately after the instruction encoding chapter. They provide readers with a high-level view of how the ISA is organized before presenting individual instructions.

# 1. Primary Opcode Map

| 0x0 Arithmetic | 0x1 Logic | 0x2 Shift/ Rotate | 0x3 Multiply/ Reduction |
| --- | --- | --- | --- |
| 0x4 Load | 0x5 Store | 0x6 Branch/ Control | 0x7 Compare |
| 0x8 Packed DSP | 0x9 SIMD / Packed | 0xA Streams | 0xB System |
| 0xC Control / Special | 0xD Mode | 0xE Extensions | 0xF Reserved |

# 2. Opcode Space Allocation (Conceptual)

Approximate emphasis of the Aurora ISA. Final percentages track the frozen opcode database.

| Instruction Family | Relative Allocation |
| --- | --- |
| Integer Arithmetic | Largest |
| Load / Store | Large |
| Packed DSP | Large |
| Logic | Medium |
| Branch / Control | Medium |
| Streams | Medium |
| System | Small |
| Reserved / Future | Remaining |

# 3. Decoder Organization

Instruction Word
        │
        ▼
  Primary Opcode
        │
 ┌──────┼──────┐
 ▼      ▼      ▼
Arithmetic Logic Memory
        │
    Sub-opcode
        │
 ADD  SUB  CMP  ...

# 4. Instruction Family Index

| Family | Primary Opcode | Functional Unit | Pipeline | Reference Chapter |
| --- | --- | --- | --- | --- |
| Arithmetic | 0x0 | Integer ALU | EX/MEM | 5 |
| Logic | 0x1 | Integer ALU | EX/MEM | 6 |
| Shift/Rotate | 0x2 | Shifter | EX/MEM | 7 |
| Multiply/Reduction | 0x3 | Shared Arithmetic | EX/MEM | 8 |
| Load/Store | 0x4-0x5 | LSU | EX/MEM | 9 |
| Branch | 0x6 | Branch Unit | EX/MEM | 10 |
| Streams | 0xA | Stream Engine | EX/MEM | 11 |
| System | 0xB-0xD | Control | WB/RET | 12 |

# 5. ISA Expansion Policy

Reserved opcode regions remain unused until approved through an Architecture Decision Record.

Instruction families are kept contiguous where practical to simplify decoder logic.

Documentation reflects the canonical frozen ISA database.