AURORA

Memory Scoreboard and Variable-Latency Completion

Phase 2 Microarchitecture Workbook — Version 1.0

# 1. Executive Summary

Aurora uses a compact scoreboard primarily to allow memory operations to outlive the nominal four-stage pipeline. Multiply and divide reuse the same dependency machinery, but the design is optimized around loads, stores, stream accesses, and memory backpressure. The core remains in-order issue and in-order retirement; there is no renaming or general dynamic scheduling.

# 2. Design Goals

Allow independent instructions to continue while a load, store, stream access, multiply, or divide remains outstanding.

Stall exactly on true register, flag, stream-state, ordering, or structural dependencies.

Preserve precise exceptions and program-order store visibility.

Keep hardware substantially smaller than a reorder buffer or reservation-station design.

Support deterministic worst-case timing for MCU and real-time DSP software.

# 3. Frozen Baseline Organization

The baseline uses an eight-entry completion queue. Entries are allocated in issue order and retired strictly from the head. The queue is not a reorder buffer: it stores only the metadata needed to track unfinished operations and precise completion.

| Field | Width / form | Purpose |
| --- | --- | --- |
| Valid / done / fault | 1 bit each | Entry lifecycle and precise exception state |
| Destination register | 4 bits + write-enable | Register dependency and writeback |
| Destination pair | 1 bit | Marks even/odd 64-bit result |
| Flags write mask | NZCVQ mask | Pending status dependencies |
| Operation class | Memory / compressor / divide / stream / system | Completion routing and serialization |
| Sequence age | Queue position | In-order retirement; no associative age compare required |
| Memory attributes | Load/store, size, bank, device, ordering | Ordering and fault handling |
| Stream identifier | R0 / R4 / coupled LIFO / none | Hidden-state dependency tracking |
| Result tag | Small unit-local tag | Matches returning data to the queue entry |

# 4. Register Busy Tracking

A 16-bit register-busy vector is derived from non-retired queue entries. Each register also stores the producing queue index so returning results and forwarding can be matched without a content-addressable reservation station.

A 32-bit destination marks one register busy.

A 64-bit result marks the named even register and implied odd register busy.

An instruction stalls in ID/RR if any source register is busy and its value is not available through an approved forwarding path.

WAW hazards stall because there is no register renaming.

WAR hazards do not arise after operands are captured in ID/RR, but a paired instruction is rejected if same-cycle semantics would conflict.

# 5. Memory Operation Tracking

Four of the eight completion entries may be outstanding memory-class operations. This limit bounds load/store queues and memory-side tags while leaving room for arithmetic operations.

Loads may complete out of execution order but write architectural registers only when their queue entry reaches retirement; forwarding may expose completed load data to younger dependent instructions after all older fault conditions are known.

Stores capture address, data, byte enables, and fault status in a two-entry store buffer. They become globally visible in program order.

An unresolved older store blocks a younger load when addresses may alias. If both addresses and byte enables are known and non-overlapping, the load may proceed.

Memory-mapped device accesses are serializing and do not bypass older memory operations.

Bank conflicts delay acceptance but do not require extra architectural rules.

A bus fault marks the owning entry faulted; no address, count, register, or flag update retires for that instruction.

# 6. Stream Dependencies

R0 and R4 each have a stream-busy bit associated with an outstanding architectural stream access.

Reconfiguration, synchronization, flush, enable/disable, and coupled-LIFO transitions wait for all older accesses to the affected stream.

Independent R0 and R4 accesses may overlap.

A scalar use of R0 or R4 conflicts with a pending stream write to the same architectural register.

Address/count advancement is committed with the owning instruction at retirement, while prefetch-buffer movement remains restart-safe implementation state.

# 7. Flags and Branches

Pending NZCV producers are tracked by the oldest not-yet-retired flag-writing entry.

A branch stalls until the NZCV version it consumes is complete and architecturally safe.

Sticky Q may be accumulated in completed entries but is ORed into PSR only at retirement.

PSR writes, synchronization operations, calls that require precise memory state, and exception return are serializing.

# 8. Issue, Completion, and Retirement

| Event | Allowed condition | Action | Stall cause |
| --- | --- | --- | --- |
| Issue | Queue entry available; sources/resources legal | Allocate one or two adjacent entries in program order | Full queue or hazard |
| Unit completion | Matching result tag returns | Mark done; capture result/fault | Completion port busy |
| Forward | Result done and no older exception can invalidate use | Provide result to ID/RR or EX/MEM | Result not ready |
| Retire | Head entry done and not blocked by older pair semantics | Commit register/flags/stream/store effects | Head not done |
| Exception | Head entry faulted | Discard younger entries; vector precisely | None once at head |

# 9. Completion Ports

The baseline has two completion-result paths: one memory/stream return path and one arithmetic path shared by the compressor engine and divider. If two units complete on the same path in one cycle, the lower-priority unit holds its valid result until accepted. Completion is decoupled with valid/ready handshakes.

# 10. Precise Exception Rules

Only the head entry may create an architectural exception.

Younger completed results remain buffered and are discarded on an older fault.

Stores do not become visible before all older potentially faulting instructions are safe.

Interrupts are accepted only at a retirement boundary with no partially retired dual-issue pair.

A prefix/consumer instruction group occupies one logical retirement unit.

# 11. Gate-Cost and Timing Expectations

Planning estimate: eight compact metadata entries, a 16-register producer table, small flag/stream busy state, two completion arbiters, and a two-entry store buffer. The likely critical path is ID/RR hazard checking, not queue storage. The design avoids full associative wakeup/select logic.

Use direct register-indexed producer tags rather than scanning every queue entry for RAW hazards.

Use queue-head comparisons for retirement and store ordering.

Keep device/serialization checks in a separate slow control path when possible.

Pipeline or predecode opcode class bits in IF to reduce ID/RR pairing and scoreboard delay.

# 12. Verification Plan

Random variable memory latency with bank conflicts and bus backpressure.

Load-use, WAW, flag-use, stream-state, and structural hazard tests.

Older-fault/younger-complete precise exception tests.

Store-to-load forwarding and ambiguous-alias stalls.

Dual-issued pair retirement and fault ordering.

Queue-full, completion collision, flush, interrupt, and reset cases.

Formal proof that architectural retirement order equals program order.

# 13. Architecture Review Result

PASS — An eight-entry in-order completion queue with direct producer tags gives Aurora most of the latency-hiding benefit needed for memory and stream operations without introducing out-of-order scheduling or renaming. It is appropriately bounded for a low-cost deterministic core.