# Aurora Stream Engine Workbook

Phase 2 Microarchitecture Baseline v1

# 1. Purpose

The stream engine converts the architectural behavior of R0 and R4 into a small, deterministic hardware block that hides memory latency, performs implicit address generation, and supplies or accepts data for Slot B without turning the core into an out-of-order machine.

# 2. Frozen Architectural Inputs

R0 and R4 are 32-bit architectural stream registers when enabled.

Supported stream element widths are 8, 16, and 32 bits only.

Each stream has current byte address, signed byte stride, direction, count, enable, completion, and fault state.

Each stream owns a two-doubleword staging buffer: 128 bits because a native doubleword is 64 bits.

R0 and R4 can operate independently or couple into one 256-bit logical LIFO buffer.

A successful access advances address and decrements finite count; stalled or faulting accesses do neither.

The core has a four-stage pipeline with scoreboarding focused primarily on variable-latency memory operations.

# 3. Chosen Physical Organization

Use two identical stream-engine instances, Stream 0 for R0 and Stream 1 for R4. Each instance contains a 128-bit circular staging buffer, configuration/state registers, a request generator, an element pack/unpack unit, and completion/fault logic.

Four 32-bit physical buffer lanes per stream.

Read and write pointers plus a 3-bit occupancy count from 0 to 4 words.

Byte-valid masks per 32-bit lane to support 8- and 16-bit elements without sub-byte tracking.

No 4-bit packing hardware.

One outstanding refill burst and one outstanding drain burst per stream.

One architectural stream element may be consumed or produced per cycle when the buffer permits.

# 4. Buffer Interpretation

| Element width | Elements per 32-bit lane | Elements per stream buffer | Notes |
| --- | --- | --- | --- |
| 8 bits | 4 | 16 | Little-endian byte order |
| 16 bits | 2 | 8 | 1-bit halfword sub-index |
| 32 bits | 1 | 4 | Direct word access |

# 5. Read-Stream State Machine

1. IDLE: stream disabled or complete.

2. ARMED: enabled; issue prefetch when occupancy is below the refill threshold.

3. REFILL: one burst request is outstanding.

4. READY: at least one configured element is available for R0/R4 consumption.

5. DRAINED: finite count reached zero and no more elements may be consumed.

6. FAULT: precise memory or configuration fault; no further advancement until software action.

A read of an enabled stream register completes from the head element. The engine then advances the sub-index or word pointer, updates address/count at retirement, and may trigger refill.

# 6. Write-Stream State Machine

1. IDLE: stream disabled or complete.

2. ARMED: accepts produced elements while capacity remains.

3. BUFFERING: packs 8- or 16-bit elements into a 32-bit lane.

4. DRAIN: emits full or explicitly flushed lanes toward memory.

5. FLUSH: completes any partially filled lane using byte enables.

6. COMPLETE: finite count reached zero and all writes are globally visible.

7. FAULT: precise bus or configuration fault; failed element remains pending.

# 7. Refill and Drain Policy

Read refill low-water mark: 2 of 4 words occupied.

Read refill target: fill to 4 words.

Write drain high-water mark: 2 of 4 words occupied.

Write drain target: drain until 1 or 0 words remain, depending on arbitration.

Maximum stream burst: 4 doublewords / 8 native words as already frozen, but a stream instance normally requests no more than its remaining 4-word local capacity.

The controller may split requests at protection, device, or bank boundaries.

# 8. Address Generation

Each stream uses a 32-bit byte address and a signed 32-bit byte stride. The address generator computes the next architectural address in parallel with element transfer.

Positive, zero, and negative stride are supported.

Default stride equals element size.

The architectural address advances once per completed element, not once per refill word.

For packed sub-word elements, the memory byte address still follows the configured stride exactly.

No modulo/circular addressing in the baseline; this remains a future extension candidate.

# 9. Slot B and Register-File Interface

When stream mode is disabled, R0/R4 are read or written through the normal register file.

When enabled, operand decode routes R0/R4 accesses to the stream engine.

A read stream supplies a zero-extended 8- or 16-bit value or a full 32-bit value.

Signed extension is performed by the consuming instruction, not by persistent stream state.

A write stream accepts the low 8, 16, or 32 bits of the source operand.

The scoreboard marks the relevant stream state busy until the element retires.

# 10. Memory and Scoreboard Integration

Each refill/drain request receives a memory-operation tag.

The memory scoreboard tracks stream identity, direction, destination buffer space, and fault status.

Read returns may fill the staging buffer before the consuming instruction retires.

Architectural address/count changes occur only at in-order retirement of the stream access.

Write data may enter the staging buffer at execution, but global visibility remains ordered through the store/memory system.

A stream fault blocks later accesses to that stream and retires precisely at the faulting instruction.

# 11. Independent R0/R4 Operation

The two stream engines are independent and may issue memory requests concurrently. They share the memory arbiter but not buffer pointers or address state.

A single Slot B DSP operation may consume both R0 and R4 in one cycle.

Both reads succeed only if each stream has a ready element.

If either stream is empty, the paired DSP operation stalls before consuming either operand.

The two architectural accesses retire atomically with the consuming instruction.

# 12. Coupled LIFO Mode

Coupled mode joins the two 128-bit buffers into one 256-bit logical LIFO while retaining two physical banks. R0 is the architectural data/top register; R4 stream access is illegal.

Use one 32-bit stack pointer/current address and one element-width field.

PUSH: decrement pointer by element size, enqueue value, then commit pointer update at retirement.

POP: return top value, then increment pointer at retirement.

Newest elements remain in local storage; older elements spill to closely coupled SRAM.

Spill/fill ordering is strict and uses the same memory tags and precise-fault rules as streams.

Mode entry requires both engines synchronized, no outstanding requests, and buffers empty or explicitly preserved by the mode-transition sequence.

# 13. Interrupt and Context Behavior

Interrupts occur only between retired architectural accesses.

Buffer contents remain implementation state but must be restart-safe.

Normal handlers that do not touch active stream state need not flush the buffers.

Handlers that reconfigure R0/R4 save and restore the complete architectural stream state.

RTOS task switching synchronizes streams before saving state.

# 14. Faults and Corner Cases

| Condition | Result | Architectural update |
| --- | --- | --- |
| Read underflow | Instruction stalls | None |
| Write buffer full | Instruction stalls | None |
| Bus fault | Precise exception | No address/count update |
| Direction misuse | Illegal stream-operation fault | None |
| Count reaches zero | Set complete and disable | After final successful element |
| Misaligned 16/32 access | Precise alignment fault | None |

# 15. Timing and Critical Paths

The intended one-cycle stream-consumption path is: decode/intercept → ready check → buffer select → element extraction → Slot B operand latch. Address/count updates are calculated in parallel and committed later.

Keep memory-request generation off the stream-consumption critical path.

Use a shallow 4-word mux per engine.

Predecode element width to select byte/halfword/word extraction.

Use registered refill/drain decisions.

Treat dual-stream availability as a simple AND of per-engine ready signals.

# 16. Power and Area Strategy

Clock-gate each stream engine independently when disabled.

Clock only active buffer lanes and pointer state.

Gate write packing logic for 32-bit streams.

Gate unused byte-lane extraction logic according to element width.

The two small buffers use flops or latch arrays in the baseline; SRAM macros are unnecessary at this size.

# 17. RTL Decomposition

aurora_stream_pair.sv — top-level R0/R4 coordination and coupled mode

aurora_stream_engine.sv — one independent stream instance

aurora_stream_buffer.sv — 4×32 data array, byte valids, pointers, occupancy

aurora_stream_agu.sv — address/stride/count logic

aurora_stream_pack.sv — 8/16/32 extraction and insertion

aurora_stream_memif.sv — request/response and burst handling

aurora_lifo_ctrl.sv — coupled spill/fill and top-of-stack behavior

# 18. Verification Plan

All widths, positive/zero/negative strides, finite and unbounded counts.

Random refill/drain backpressure.

Simultaneous R0/R4 consumption with one or both empty.

Precise bus faults on every beat position.

Mode transitions with outstanding requests forbidden.

Coupled LIFO spill/fill across buffer boundaries.

Interrupt entry after every possible completed element.

Formal proof that address/count never advance on stall or fault.

Formal proof that no element is duplicated or dropped.

# 19. Frozen Baseline

Aurora Phase 2 uses two independent 128-bit stream buffers with 8/16/32-bit element packing, one outstanding refill and one outstanding drain per engine, deterministic low/high-water policies, scoreboarded precise memory interaction, and optional 256-bit coupled LIFO operation.