AUR-RTL-002C Register File & Scoreboard

Implementation-Ready RTL Blueprint v1.2

# 1. Purpose

Provide architectural register storage, reads, writes, pair aliasing, whole-register L8/L16 merge semantics, and busy/completion tracking for all outstanding operations.

# 2. Register File Organization

16 x 32-bit architectural GPRs.

Two combinational or registered read ports minimum; two architectural writebacks per cycle target.

R13 is LR plus conditional GPR; R14 SP; R15 PC.

R8/R10 expose accumulator low 32-bit views.

Paired writes update both halves atomically.

# 3. Ports

| Signal | Dir | Width | Meaning |
| --- | --- | --- | --- |
| ra0_i, rb0_i, ra1_i, rb1_i | in | 4x4 | Source register indices for two issued instructions |
| rdata_*_o | out | 4x32 | Read data |
| wb0_valid_i/wb1_valid_i | in | 2 | Writeback valids |
| wb*_rd_i | in | 4 | Destination register |
| wb*_data_i | in | 32 | Writeback data |
| wb*_mask_i | in | 4 | Byte write mask for L8/L16/full writes |
| pair_wb_i | in | pair bundle | Atomic paired result |
| busy_set_i/busy_clear_i | in | resource masks | Scoreboard updates |
| busy_o | out | resource masks | Current hazards |

# 4. Write Arbitration

Two unrelated scalar writes may commit together.

A paired write consumes both architectural write slots unless implemented with a dedicated pair port.

If both writes target the same register, the older architectural write wins first; younger commit waits.

L8/L16 writes read old destination and merge by byte mask before architectural commit.

R15 writes only from control/retirement path; ordinary GPR write attempts trap or are blocked.

# 5. Scoreboard Resources

| Resource | Storage | Rule |
| --- | --- | --- |
| GPR busy | 16 bits | One per register |
| Pair busy | derived | OR of both halves; writes reserve both |
| A0/A1 | 2 bits | Accumulator result not ready |
| Flags | 1 bit | PSR arithmetic flags pending |
| Q0/Q1 | 2 bits | Stream state/data pending |
| LSU/MUL/RED/DIV | unit state | Outstanding shared resource |

# 6. Merge Semantics

full: new = result
L16:  new = (old & 0xFFFF0000) | (result & 0x0000FFFF)
L8:   new = (old & 0xFFFFFF00) | (result & 0x000000FF)

# 7. Forwarding

EX result may bypass next-cycle reads.

Completion-bus results bypass once ready.

Pair forwarding presents both halves atomically.

Partial writes forward the merged 32-bit value, never an isolated lane.

# 8. Assertions

No architectural register changes without a committed write.

R14/R15 cannot be paired destinations.

Pair busy implies both halves unavailable.

Partial write preserves all masked-off bytes.

Simultaneous writes cannot create ambiguous final state.