AUR-RTL-002D Symmetric Execution Lanes

Implementation-Ready RTL Blueprint v1.2

# 1. Purpose

Execute ordinary scalar, packed, low-lane, shift, compare, move and layout operations in either of two symmetric lanes.

# 2. Lane Capabilities

| Class | Operations |
| --- | --- |
| Scalar ALU | ADD/SUB/AND/OR/XOR/MIN/MAX/CMP |
| Packed ALU | 4x8 and 2x16 lane-isolated operations |
| Low-lane | L8/L16 read-modify-write |
| Shift/rotate | Scalar and packed; paired-64 dispatched to multi-cycle helper |
| Layout | MOV, BSWAP, WSWAP, PACK, UNPACK, ZIP, UNZIP, SHUFFLE |
| Control assist | Branch compare and target assist |

# 3. Ports

| Signal | Dir | Width | Meaning |
| --- | --- | --- | --- |
| op_i | in | decoded | Operation and mode |
| src_a_i/src_b_i | in | 32 each | Operands |
| old_dst_i | in | 32 | Merge source for L8/L16 |
| carry_i | in | 1 | Carry/borrow input |
| result_o | out | 32 | Result |
| flags_o | out | ZNVC | Flags |
| branch_taken_o | out | 1 | Condition result |
| shared_req_o | out | bundle | Request to multiplier/LSU/reduction/etc. |

# 4. Packed Arithmetic

Packed modes suppress carries between lanes.

Signedness is instruction/subfunction controlled.

Horizontal reductions do not execute in general lanes; they dispatch to reduction cluster.

L8/L16 modify only low lane and preserve upper storage.

# 5. Lane Symmetry

Either lane accepts any ordinary operation. Shared-resource dispatch is lane-independent. Lane numbering has no architectural meaning.

# 6. Timing Targets

| Operation | Latency | Initiation |
| --- | --- | --- |
| ALU/logic/compare | 1 | 1 |
| Packed ALU | 1 | 1 |
| Layout/shuffle | 1 | 1 |
| Scalar shift | 1 target | 1 |
| L8/L16 merge | 1 | 1 |

# 7. Assertions

Same opcode and operands produce identical result in either lane.

Packed carries never cross lane boundaries.

Layout operations ignore lane mask.

L8/L16 preserve upper bits exactly.

Shared request includes complete age/destination metadata.