AUR-RTL-002E Multiplier & Reduction Cluster

Implementation-Ready RTL Blueprint v1.2

# 1. Purpose

Implement scalar/packed multiplication, MAC/MAS, DOT, SAD and generalized reductions using a shared Dadda-style compressor infrastructure and two 40-bit accumulators.

# 2. Internal Blocks

| Block | Role |
| --- | --- |
| Partial-product generator | Scalar/packed signed or unsigned products |
| Radix-4 step logic | Multi-cycle wide multiplication support |
| Compressor tree | 3:2 default; selective 4:2 only when synthesis wins |
| Late injection points | Four 64-bit-aligned injection points for MAC/DOT/SAD/reduce |
| Final adder | Produces scalar/packed result |
| Accumulator merge | A0/A1 overwrite/fold behavior |

# 3. Supported Operations

MUL/MULH scalar and paired support

Packed 4x8 and 2x16 multiply

MAC and MAS

DOT and SAD

REDUCE.ADD/AND/OR/XOR/MIN/MAX/ABSMAX/COUNT

Overwrite or operator-specific fold into A0/A1

# 4. Ports

| Signal | Dir | Width | Meaning |
| --- | --- | --- | --- |
| req_valid_i | in | 1 | Launch request |
| req_op_i | in | enum | Operation |
| req_mode_i | in | 3 | Packed/scalar mode |
| src0_i/src1_i | in | 64 max logical | Operands or pair bundle |
| acc_sel_i | in | 1 | A0/A1 |
| fold_i | in | 1 | Overwrite vs fold |
| consume_i | in | 2 | Stream consume override metadata |
| result_valid_o | out | 1 | Completion |
| result_o | out | 64/40 | Result/accumulator value |
| fault_o | out | 1 | Illegal mode/operation |

# 5. Pipeline

| Stage | Work |
| --- | --- |
| M0 | Decode mode, sign, lane widths; generate partial products |
| M1 | Initial Dadda compression |
| M2 | Late injections, final compression and add |
| M3 optional | Accumulator merge / paired writeback where timing requires |

# 6. Resource Rules

One reduction-class launch per cycle baseline.

Multiplier initiation interval target is one where no conflicting reduction launch exists.

DOT/SAD and REDUCE share final reduction resources.

A0 and A1 are independently scoreboarded; two dependent folds to same accumulator cannot overlap illegally.

# 7. Assertions

Reduction result equals mathematical lane operation with specified widening.

Fold uses operator-specific identity and combination.

Accumulator never loses guard bits except through explicit extraction/shift semantics.

Stream consume occurs exactly once per committed launch.

Illegal packed mode combinations fault precisely.