AUR-RTL-002G Load/Store Unit

Implementation-Ready RTL Blueprint v1.2

# 1. Purpose

Generate effective addresses, enforce alignment and protection, launch ordered memory transactions, and return scalar or paired data through the scoreboard/completion system.

# 2. Ports

| Signal | Dir | Width | Meaning |
| --- | --- | --- | --- |
| req_valid_i | in | 1 | Memory request |
| req_load_i/req_store_i | in | 1 each | Operation type |
| base_i | in | 32 | Base address |
| offset_i | in | signed immediate | Offset |
| store_data_i | in | 64 max | Store data or pair |
| width_i | in | 2 | 8/16/32/64 |
| device_i | in | 1 | MMIO/strongly ordered |
| mem_valid_o | out | 1 | Fabric request |
| mem_ready_i | in | 1 | Fabric response |
| mem_addr_o | out | 32 | Effective address |
| mem_wdata_o | out | 64 max | Write data |
| mem_rdata_i | in | 64 max | Read data |
| fault_o | out | enum | Alignment/access/bus fault |
| complete_o | out | bundle | Age-tagged completion |

# 3. Address Generation

effective_address = (base + sign_extend(offset)) mod 2^32

SP/PC sources are ordinary 32-bit addresses.

Paired-64 data width does not create a 64-bit address space.

Pair loads/stores require 8-byte alignment in portable ABI code.

Unaligned support is profile-defined; baseline may trap to ASC.

# 4. Memory Classes

| Class | Ordering | Speculation |
| --- | --- | --- |
| TCM/SRAM normal | Stores ordered; loads conservative | No load-over-store disambiguation baseline |
| External normal | Same architectural order; may wait | Outstanding request scoreboarded |
| MMIO/device | Strongly ordered | Never speculative |
| Instruction synchronization | Explicit barrier after executable writes | Required |

# 5. Bank Selection

General SRAM interleaves at native 16-bit word granularity.

LSU presents address and byte strobes; fabric selects bank.

Bank conflicts stall or serialize deterministically.

DMA and streams arbitrate through the fabric; CPU retirement-critical traffic has bounded service.

# 6. Store Commit Rule

A store may not become externally visible until it reaches its architectural retirement point. MMIO writes are never canceled after acceptance.

# 7. Assertions

No duplicate store after interrupt or branch recovery.

MMIO order matches program order.

Pair load becomes visible atomically.

Faulting load/store produces no partial architectural destination update.

Byte strobes match width and address.