AUR-RTL-002A Fetch & Branch Predictor

Implementation-Ready RTL Blueprint v1.2

# 1. Purpose

Fetch up to two adjacent 16-bit words per cycle, assemble extension-word sequences, predict simple control flow, and redirect fetch on branch, exception, or interrupt recovery.

# 2. Ports

| Signal | Dir | Width | Meaning |
| --- | --- | --- | --- |
| clk_i | in | 1 | Core clock |
| rst_i | in | 1 | Synchronous reset |
| redirect_valid_i | in | 1 | Branch/exception/interrupt redirect |
| redirect_pc_i | in | 32 | Redirect target |
| fetch_addr_o | out | 32 | Instruction memory address |
| fetch_req_o | out | 1 | Fetch request valid |
| fetch_ready_i | in | 1 | Instruction memory accepts request |
| fetch_data_i | in | 32 | Two adjacent 16-bit words |
| fetch_data_valid_i | in | 1 | Returned fetch data valid |
| fetch_fault_i | in | 1 | Instruction access fault |
| decode_ready_i | in | 1 | Decode window can accept words |
| word0_o/word1_o | out | 16 each | Fetched words |
| pc0_o/pc1_o | out | 32 each | Associated PCs |
| valid_o | out | 2 | Word valid bits |

# 3. Predictor

Backward conditional branches are predicted taken.

Forward conditional branches are predicted not taken.

Unconditional direct branches are predicted taken when target formation is available.

No branch target buffer is required in the v1.2 baseline.

Prediction state is combinational from decoded direction and immediate sign; no history table is required.

# 4. Prefetch Buffer

Single predicted path only.

Minimum capacity: four 16-bit words beyond the current decode demand.

Redirect invalidates all prefetched words younger than redirect point.

Extension words remain associated with their base PC and are never independently retired.

# 5. Fetch State Machine

IDLE:
    if decode_ready: issue fetch at fetch_pc
WAIT:
    if redirect: cancel/kill response if possible; set fetch_pc = redirect_pc
    elif response_valid:
        enqueue returned words
        fetch_pc += number_of_valid_words * 2
DELIVER:
    present up to two adjacent words to decode

# 6. Assertions

PCs increase by two bytes for adjacent 16-bit words.

Redirected wrong-path words never reach decode as valid.

Extension words retain original fetch order.

Instruction faults report the address of the failing fetch.

At most two words are delivered per cycle.