AUR-RTL-002F DIVSTEP Assist & Division Runtime Interface

Corrected Implementation-Ready RTL Blueprint v1.2a

# 1. Architectural Decision

Aurora v1.2 does not implement a large dedicated integer divider. Division is provided by the architectural DIVSTEP primitive, which reuses the existing execution-lane ALU, shifter, compare/subtract, carry/borrow, and register-pair datapaths. Full divide and remainder operations are synthesized by the compiler/runtime or implemented through an ASC helper.

# 2. Purpose of This Module

Provide the small amount of control and operand-routing logic needed to execute one divide iteration. This block is an assist unit, not a standalone quotient/remainder engine.

# 3. Explicit Non-Goals

No dedicated 32-bit or 64-bit quotient/remainder datapath.

No wide normalization tree.

No multi-stage divide pipeline.

No separate divider forwarding network.

No requirement to keep an entire divide operation internally outstanding.

No extra architectural divider state beyond normal registers, flags, and optional loop state.

# 4. Reused Hardware

| Existing resource | DIVSTEP use |
| --- | --- |
| General execution lane ALU | Conditional subtract/add of divisor from partial remainder |
| Shifter/rotate path | Shift combined remainder/quotient state by one or two bits |
| Compare/carry logic | Select quotient bit(s) and next remainder |
| Register pairs | Hold dividend, quotient, and partial remainder state |
| Flags/condition logic | Expose carry/borrow/sign decisions |
| Low-overhead loop support | Repeat the fixed number of DIVSTEP iterations |

# 5. Architectural Operation

A DIVSTEP instruction transforms software-visible divide state by one bounded iteration. The exact register convention is defined by the ISA reference and runtime ABI.

conceptual unsigned radix-2 step:

combined = (remainder : quotient) << 1
trial     = combined.remainder - divisor

if trial >= 0:
    remainder = trial
    quotient[0] = 1
else:
    remainder = combined.remainder
    quotient[0] = 0

A radix-4 implementation may generate two quotient bits per instruction if synthesis shows the small additional selection logic is worthwhile. The ISA semantics must remain deterministic and profile-visible.

# 6. Suggested Ports

| Signal | Dir | Width | Meaning |
| --- | --- | --- | --- |
| req_valid_i | in | 1 | DIVSTEP request |
| req_signed_i | in | 1 | Signed/unsigned step mode |
| req_radix_i | in | 1 or profile | Radix-2 or radix-4 internal choice |
| state_hi_i | in | 32 | Partial remainder/high state |
| state_lo_i | in | 32 | Quotient/dividend low state |
| divisor_i | in | 32 | Divisor |
| result_hi_o | out | 32 | Updated partial remainder |
| result_lo_o | out | 32 | Updated quotient/dividend state |
| step_flags_o | out | small | Quotient bit(s), borrow/sign metadata |
| complete_o | out | 1 | One step completed |
| illegal_o | out | 1 | Illegal mode or register form |

# 7. Compiler and Runtime Expansion

C integer division and remainder lower to a runtime helper or an inline DIVSTEP loop.

The compiler may choose inline expansion for constant or hot divisors.

The ASC runtime provides canonical signed and unsigned 32-bit helpers.

Paired-64 division is a software routine using paired shifts, subtracts, and DIVSTEP-compatible control.

A DIV mnemonic may remain as an assembler pseudo-instruction only.

# 8. Exceptional Cases

| Case | Handling |
| --- | --- |
| Divide by zero | Detected before the step loop by compiler/runtime or minimal setup logic; precise fault or language-defined helper result |
| Signed MIN / -1 overflow | Detected by signed helper setup; precise architectural behavior |
| Interrupt during sequence | Each DIVSTEP is independently retired; interruption occurs between steps |
| Fault/restart | No hidden multi-cycle divide state exists, so restart is ordinary instruction restart |

# 9. Timing and Area Targets

| Property | Target |
| --- | --- |
| DIVSTEP latency | 1 cycle preferred; 2 cycles acceptable if routed through shared shift/subtract path |
| Initiation interval | 1 |
| Dedicated state | Minimal or none beyond request/result registers |
| Area objective | Small control/mux increment relative to one general execution lane |
| Power objective | Clock-gated except during DIVSTEP |

# 10. Pairing and Scheduling

DIVSTEP may issue in either symmetric execution lane.

It consumes ALU, shifter, compare, and flag resources for that cycle.

It may pair with an independent instruction only when the second lane and shared register-file/writeback resources are available.

A sequence of DIVSTEP instructions is compiler-scheduled like any other dependent loop.

The four-entry issue window may pair independent loop bookkeeping or address work around the sequence.

# 11. Assertions

One DIVSTEP changes only its specified destination state and flags.

No hidden divide state remains after retirement.

Canceled or flushed DIVSTEP produces no architectural write.

Unsigned step result matches the specified subtract/shift recurrence.

Signed helper sequences produce the language-defined quotient and remainder.

A DIV pseudo-instruction never reaches RTL as a distinct hardware opcode.

# 12. Verification Plan

| Test class | Method |
| --- | --- |
| Single-step recurrence | Exhaustive reduced-width model + random 32-bit |
| Full unsigned divide | Runtime loop vs mathematical reference |
| Full signed divide | All sign combinations and corner cases |
| Divide by zero | Directed precise-fault tests |
| Interrupt between steps | Directed retirement-boundary injection |
| Dual-issue interaction | Constrained-random resource conflict tests |

# 13. Superseded Wording

This document supersedes earlier AUR-RTL-002F text describing a self-contained iterative divider with internal quotient, divisor, remainder, and iteration state. Those structures are not part of the Aurora v1.2 baseline.