AUR-RTL-002F Divider

Implementation-Ready RTL Blueprint v1.2

# 1. Purpose

Implement integer division using an iterative radix-step engine, supporting signed/unsigned scalar operation and paired-64 assistance without a full single-cycle divider.

# 2. State

| Field | Meaning |
| --- | --- |
| busy | Operation active |
| signed | Signed or unsigned |
| dividend | Working dividend/remainder |
| divisor | Normalized divisor |
| quotient | Partial quotient |
| step | Iteration counter |
| age/destination | Completion metadata |
| fault | Divide-by-zero/overflow |

# 3. Algorithm

Baseline may use radix-2 or radix-4 restoring/non-restoring division.

DIVSTEP exposes one architectural step for software-assisted forms.

Full DIV runs internally until quotient/remainder complete.

Signed division converts operands to magnitude and restores signs at completion.

# 4. Interruptibility

Divider should provide checkpoint/abort between iterations.

If canceled for precise interrupt, the whole instruction restarts.

Maximum non-abortable interval should be one iteration.

One outstanding divide is sufficient for v1.2.

# 5. Assertions

quotient * divisor + remainder equals dividend for valid operations.

Remainder magnitude is less than divisor magnitude.

Divide-by-zero produces precise fault and no destination write.

Canceled divide produces no architectural result.

Signed overflow case is explicitly defined.