AUR-RTL-002H Stream Engines Q0/Q1

Implementation-Ready RTL Blueprint v1.2

# 1. Purpose

Implement two independent stream engines over R0 and R4, each with address, stride, width, count, circular addressing, consume/peek behavior, and a 128-bit staging buffer.

# 2. Per-Stream State

| Field | Width | Meaning |
| --- | --- | --- |
| enabled | 1 | Stream interception active |
| address | 32 | Current byte address |
| stride | 32 signed | Byte stride after consume/produce |
| width | 2 | 8/16/32-bit element |
| count | 32 | Remaining elements |
| limit/base | 32 each | Circular buffer parameters |
| direction | 1 | Read or write stream |
| head/tail | index | Staging-buffer pointers |
| fill | 3 | Number of valid 32-bit lanes |
| end_mode | enum | Stop, wrap, signal, predicate |

# 3. Ports

| Signal | Dir | Width | Meaning |
| --- | --- | --- | --- |
| req_valid_i | in | 1 | Stream operation request |
| stream_sel_i | in | 1 | Q0/Q1 |
| op_i | in | enum | peek/pop/push/dup/replace/config |
| consume_override_i | in | 1 | Consume vs local register access |
| src_data_i | in | 64 max | Produced value |
| result_o | out | 64 max | Consumed/peeked value |
| result_valid_o | out | 1 | Result ready |
| mem_req_o | out | bundle | Refill/spill request |
| mem_rsp_i | in | bundle | Memory response |
| state_busy_o | out | 2 | Q0/Q1 busy |

# 4. Buffer Behavior

Each engine has a 128-bit staging buffer organized as four 32-bit lanes.

8-bit and 16-bit elements pack within lanes; lane extraction is width-aware.

Coupled LIFO mode may combine Q0 and Q1 into one 256-bit logical stack.

Peek returns the next element without advancing address/count.

Simultaneous pop and push replacement may update the top element without changing depth.

# 5. Circular Addressing

Circular mode requires aligned power-of-two regions in the baseline. The next address masks selected low carry bits while preserving the aligned base.

next = base | ((address - base + stride) & (size - 1))

# 6. Ordering and Hazards

Consumption remains ordered independently for Q0 and Q1.

Register-only MOV/PACK/UNPACK/SHUFFLE operations do not consume stream elements.

Configuration changes serialize later operations on the affected stream.

A stream fault must not partially advance architectural address/count state.

Q0 and Q1 may refill concurrently when memory-fabric capacity permits.

# 7. Assertions

Peek never changes address, count, or fill.

Consume advances exactly once per committed operation.

Circular address always remains inside configured region.

Buffer fill remains between zero and four lanes.

Faulting transfers leave stream state restartable.