AUR-RTL-002I Memory Fabric

Implementation-Ready RTL Blueprint v1.2

# 1. Purpose

Arbitrate instruction fetch, LSU, Q0/Q1, DMA, TCM, banked SRAM, MMIO, and external-memory transactions with deterministic ordering and bounded service.

# 2. Masters and Slaves

| Type | Interfaces |
| --- | --- |
| Masters | IF, LSU, Q0, Q1, DMA channels, debug |
| Slaves | ITCM, DTCM, SRAM bank 0/1, MMIO, external SRAM/flash/DRAM |

# 3. Request Bundle

| Field | Width | Meaning |
| --- | --- | --- |
| valid | 1 | Request valid |
| master_id | enum | Origin |
| address | 32 | Byte address |
| write | 1 | Write/read |
| wdata | 64 max | Write payload |
| wstrb | 8 max | Byte strobes |
| size | 3 | Transfer width |
| device | 1 | Strong-order attribute |
| age | tag | Retirement/completion tag |

# 4. Arbitration Policy

TCM requests use dedicated paths where implemented.

CPU retirement-critical requests have bounded priority.

Stream refill may receive deterministic reservation slots.

DMA receives remaining burst opportunities but cannot starve CPU/streams.

MMIO is non-bursting and strongly ordered.

Arbitration policy is fixed or privileged-configured and must be documented.

# 5. SRAM Banking

Bank selection uses native 16-bit word interleaving.

Two independent banks permit concurrent accesses when addresses select different banks.

Same-bank conflicts serialize deterministically.

Instruction fetch and data access may proceed concurrently if mapped to different banks or dedicated TCM.

# 6. Response Rules

Responses return master ID, age tag, data, ready and fault.

Responses may complete out of order only if the requesting master supports tags.

MMIO responses preserve issue order.

External ready/acknowledge wait states are absorbed without changing architectural order.

# 7. Assertions

Every accepted request receives exactly one response or documented cancellation.

No master is starved beyond configured bound.

MMIO order equals request order.

Same-bank collisions never corrupt data.

Fault response does not include stale data as valid.