AUR-RTL-002K Interrupt & Timer Unit

Implementation-Ready RTL Blueprint v1.2

# 1. Purpose

Implement sixteen fixed-priority interrupt levels, source routing, pending/active state, precise retirement-boundary recognition, one coarse countdown timer, and one coarse free-running counter.

# 2. Interrupt State

| Field | Meaning |
| --- | --- |
| pending | Pending source/vector bits |
| enable | Source enable mask |
| route | Source to vector/priority mapping |
| threshold | Current minimum accepted priority |
| active | Current nesting stack or active vector |
| vector_base | Base of active vector table |

# 3. Priority Rule

Lower vector number denotes higher architectural priority.

Equal-priority sources use deterministic fixed subpriority or configured round-robin if profile permits.

Task scheduling policy such as EDF remains software.

Nested interrupts are allowed only when new priority exceeds current threshold.

# 4. Timer State

| Register | Meaning |
| --- | --- |
| CYCLE_COUNT | 32-bit coarse monotonic counter |
| TIMER_COUNT | 32-bit one-shot countdown |
| TIMER_VECTOR | Vector to pend on expiration |
| TIMER_CTRL | Enable, hold, clear, prescale |

# 5. Timer Semantics

Timer tick may represent 2^P core cycles.

Countdown decrements on timer ticks, not necessarily every core cycle.

Expiration posts the programmed vector with fixed documented latency.

Software multiplexes all OS deadlines onto the one countdown timer.

Counter implementation may be ripple/segmented/serial because it is off the critical path.

# 6. Assertions

Highest-priority enabled pending vector is selected.

Timer expiration posts exactly one pending event per load.

Disabled timer never decrements or fires.

Interrupt entry only begins at a legal retirement boundary.

Vector-base alignment is enforced.