AUR-RTL-002L Debug & Trace

Implementation-Ready RTL Blueprint v1.2

# 1. Purpose

Provide nonintrusive architectural debug, breakpoints, single-step, trace visibility, and performance-observation hooks without changing normal execution semantics.

# 2. Debug Features

Halt/resume at retirement boundary.

Single-step one architectural instruction or retirement group.

Instruction-address breakpoints.

Optional data watchpoints.

Read/write GPR, PSR, accumulators, streams and system registers while halted.

Optional trace of issue, completion, retirement, memory and interrupts.

# 3. Trace Record

| Field | Meaning |
| --- | --- |
| cycle | Coarse or exact cycle count |
| pc | Instruction PC |
| opcode | Decoded instruction |
| issue lanes | Which instructions issued |
| stall reason | Dependency/resource/memory |
| writes | Architectural destinations |
| memory | Address/data/type |
| event | Branch, fault, interrupt, DMA/stream |

# 4. Intrusiveness Rules

Disabled debug logic must not affect architectural timing except documented synthesis impact.

Trace overflow may drop records but must not stall the core unless explicitly configured.

Halting waits for precise boundary; externally visible stores are not duplicated.

Debug writes to PC/PSR invalidate prefetched and decoded state.

# 5. Assertions

Single-step retires the requested number of architectural instructions.

Breakpoint halts before executing the matching instruction unless configured after-execute.

Trace records preserve retirement order.

Debug access while running cannot corrupt register file arbitration.

Resume restarts from architecturally visible PC.