# AUR-ARCH-005 Addendum — Prefetch and Eight-Word Hardware Loops v1.2s

# Instruction Prefetch

The front end fetches 32 bits per cycle and contains two 32-bit prefetch entries. Together these entries retain four adjacent 16-bit instruction words.

# Captured Hardware-Loop Window

A captured hardware loop may contain up to eight contiguous 16-bit instruction words:

  words 0-3: decoded replay buffer
  words 4-7: pinned raw prefetch entries

Words 4-7 pass through the normal decoder on every replay but do not require instruction-memory or I-cache access.

# Instruction Boundaries

Extension words count toward the eight-word limit.

Replay begins only at a base instruction.

The final captured word must complete an instruction.

The assembler diagnoses any loop whose extension sequence crosses the eight-word boundary.

# Loop Completion and Replay

A body that uses all eight words terminates implicitly at capacity.

A shorter body uses the established loop-end marker or encoded body-length mechanism.

The loop sequencer decrements the count and selects the captured start without branch prediction.

No instruction pair may span the iteration wrap.

Normal dual-issue pairing is allowed across the word-3/word-4 storage boundary.

# Fallback

Bodies longer than eight words are not captured in the local replay window. They execute through the ordinary fetch path while retaining the architecture's loop-count and backward-loop control behavior.

# Example: Six-Instruction Argmax Loop

LOOPR Rcount
    POPMETA  R4:R5, Q0, ORD
    ABS      R4, R4
    CMP      Rbest, R4
    CMOV.LT  Rbest, R4
    CMOV.LT  Rbestidx, R5
    REDUCE.ADD A0, R4, ACC
; six one-word instructions occupy six of eight loop words