# Aurora Architecture v1.2
Architecture Bible
Design Rationale & Decision Log

Purpose: This document is the long-term engineering record for Aurora. Every major architectural decision is captured with its motivation, alternatives, tradeoffs, chosen solution, consequences, and future expansion path. It is intended to prevent design drift and preserve architectural intent.

# 1. Product Mission

## Problem

Determine where Aurora competes.

## Alternatives Considered

High-end DSP

General-purpose CPU

Traditional MCU

MCU + DSP consolidation

## Decision

Target MCU + low-to-mid-range fixed-point DSP consolidation.

## Why This Was Chosen

Most embedded systems needing meaningful DSP today combine a control processor with a DSP. Integrating both reduces board complexity while preserving deterministic behavior.

## Consequences

Lower BOM

Lower latency

Lower power

Simpler software stack

## Future Expansion

Scale memory and peripherals across Aurora-L/M/P while keeping one ISA.

# 2. ISA Width

## Problem

Balance code density and decoding simplicity.

## Alternatives Considered

16-bit fixed

32-bit fixed

Variable length

## Decision

16-bit base ISA with extension words.

## Why This Was Chosen

Excellent embedded code density without forcing all instructions to carry large immediates.

## Consequences

Small flash footprint

Simple decoder

Occasional extension-word cost

## Future Expansion

Additional extension formats can be added without changing the base ISA.

# 3. Register File

## Problem

Provide enough registers while minimizing area.

## Alternatives Considered

8 GPR

16 GPR

32 GPR

Separate address/data files

## Decision

16 unified 32-bit GPRs.

## Why This Was Chosen

Compiler-friendly and simpler than classic DSP split register files.

## Consequences

Good C performance

Lower hardware complexity

Unified programming model

## Future Expansion

Future profiles may add implementation-private shadow registers, but architectural count remains 16.

# 4. 64-bit Support

## Problem

Support 64-bit arithmetic without doubling hardware.

## Alternatives Considered

Native 64-bit core

Dedicated 64-bit register bank

Paired-register emulation

## Decision

Paired even/odd registers emulate 64-bit values.

## Why This Was Chosen

64-bit operations are relatively uncommon in the target market; spending cycles is preferable to spending silicon.

## Consequences

Higher latency

Small area

Compiler manages pairs

## Future Expansion

Selective paired instructions may be added where cost-effective.

# 5. Pipeline Philosophy

## Problem

Increase throughput without high OoO complexity.

## Alternatives Considered

Single issue

Out-of-order

Wide VLIW

Dual issue with local lookahead

## Decision

Four-stage in-order pipeline with a four-entry dynamic pairing window.

## Why This Was Chosen

Most ILP can be exposed by the compiler; hardware only repairs nearby missed opportunities.

## Consequences

Predictable timing

Small hardware

Good dual-issue utilization

## Future Expansion

Window depth can grow in future cores without ISA changes.

# 6. DSP Features

## Problem

Deliver strong fixed-point performance.

## Alternatives Considered

Conventional ALU only

Dedicated vector engine

Streams + packed SIMD + accumulators

## Decision

Two 40-bit accumulators, stream engines, packed modes, reductions, circular addressing.

## Why This Was Chosen

These features provide high sustained DSP throughput with modest hardware.

## Consequences

Excellent FIR/IIR support

Good AI preprocessing

Low power

## Future Expansion

Additional reduction operators or stream profiles remain ISA-compatible.

# 7. Memory System

## Problem

Provide deterministic bandwidth.

## Alternatives Considered

Mandatory caches

TCM only

Optional ITCM/DTCM + banked SRAM + DMA

## Decision

Optional TCM with unified address map.

## Why This Was Chosen

Many MCU-oriented products do not need TCM, while DSP-oriented products benefit greatly.

## Consequences

Flexible product scaling

Deterministic hot code/data

## Future Expansion

Future implementations may add caches transparently.

# 8. Compiler Philosophy

## Problem

Divide optimization work between software and hardware.

## Alternatives Considered

Hardware scheduling

Compiler scheduling

Hybrid

## Decision

Compiler performs primary scheduling; hardware performs local pairing repair.

## Why This Was Chosen

Compiler has global visibility while hardware only needs inexpensive local logic.

## Consequences

Simpler silicon

Strong LLVM/GCC backend importance

## Future Expansion

PGO and auto-vectorization can further improve results.

# 9. Guiding Principles

## Problem

Establish enduring engineering rules.

## Alternatives Considered

## Decision

Adopt a stable set of design principles.

## Why This Was Chosen

## Consequences

Prefer deterministic behavior over speculative peak performance.

Spend silicon on throughput, not speculation.

Reuse hardware aggressively.

Keep software portability a first-class objective.

Optimize for whole-system cost, not benchmark leadership.

Allow scalable implementation profiles without ISA fragmentation.

## Future Expansion

These principles should be consulted before any future architectural change.

# Open Questions for Future Versions

Whether a second LSU launch path is justified by synthesis results.

Whether paired-64 shift/rotate should gain additional helper instructions.

Whether optional cache profiles provide worthwhile system benefits.

Whether future AI-oriented packed integer operations merit standardization.

# Architecture Freeze

The programmer's model, ABI, ISA, pipeline philosophy, memory map, compiler model, interrupt model, and product positioning are considered frozen for Aurora v1.2. Subsequent revisions should preserve backward compatibility whenever practical.