# Aurora Architecture v1.2

Canonical Memory Map and Privileged-System Register Map

Draft 1 - August 2026

This specification defines the canonical 32-bit physical address layout, required and optional memory regions, privileged system registers, interrupt-controller window, DMA window, TCM profiles, memory attributes, and ordering rules for Aurora v1.2 implementations.

# 1. Architectural Principles

Aurora uses a unified 32-bit byte-addressed physical address space.

ITCM, DTCM, SRAM, ROM, MMIO and external memory are separate targets on the internal memory fabric.

TCM is an implementation-profile feature; the baseline ISA does not require every Aurora device to include it.

Memory-mapped I/O is strongly ordered, non-cacheable and non-executable.

The v1.2 baseline does not require L1 or L2 caches.

ASC ROM and reset services must not depend on external memory.

DMA and stream engines use the same physical address map as the CPU, subject to protection and arbitration.

# 2. Canonical Physical Address Map

| Start | End | Size | Region | Required | Attributes | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x0000_0000 | 0x0000_FFFF | 64 KiB | Boot/ASC ROM | Yes | RX, privileged write-protected | Reset entry, ASC services, recovery, immutable vectors. |
| 0x0001_0000 | 0x0001_FFFF | 64 KiB | ITCM Window | Profile-dependent | RX or RWX by profile | Deterministic instruction memory; absent regions fault or alias implementation memory only if profile says so. |
| 0x0002_0000 | 0x0002_FFFF | 64 KiB | DTCM Window | Profile-dependent | RW, executable optional | Deterministic data memory, DMA-accessible. |
| 0x0003_0000 | 0x0003_FFFF | 64 KiB | Vector/Protected RAM Window | Recommended | RW, privileged configuration | Relocatable active vector table, interrupt stacks, ASC scratch. |
| 0x0010_0000 | 0x0FFF_FFFF | 255 MiB | On-chip SRAM / implementation RAM | Yes, size varies | RWX by MPU policy | Banked SRAM, shared CPU/stream/DMA access. |
| 0x1000_0000 | 0x1FFF_FFFF | 256 MiB | Peripheral MMIO | Yes | Device, strongly ordered, non-executable | Timers, interrupt controller, DMA, serial, GPIO and platform peripherals. |
| 0x2000_0000 | 0x2FFF_FFFF | 256 MiB | External SRAM/PSRAM | Optional | RWX by MPU policy | Ready/acknowledge external-memory controller. |
| 0x3000_0000 | 0x3FFF_FFFF | 256 MiB | External Flash / XIP | Optional | RX, writes through controller | Bootable code and constants. |
| 0x4000_0000 | 0x7FFF_FFFF | 1 GiB | External DRAM / high-capacity memory | Optional | RWX by MPU policy | High-end profile; DMA and stream engines may access. |
| 0x8000_0000 | 0xEFFF_FFFF | 1.75 GiB | Platform / implementation-defined | No | Profile-defined | Accelerators, additional memory windows, SoC integration. |
| 0xF000_0000 | 0xFFFF_FFFF | 256 MiB | System control and debug | Yes | Privileged, device, non-executable | Core-local registers, debug, trace, MPU and implementation ID. |

# 3. Reset, ASC and Interrupt Vectors

After reset, instruction fetch begins in Boot/ASC ROM. The immutable reset vector and recovery entry remain available even if external flash or SRAM is unavailable.

Reset entry is at 0x0000_0000.

The immutable ROM vector table occupies the beginning of Boot/ASC ROM.

VECTOR_BASE selects the active vector table after startup.

The active vector table should normally reside in ITCM or protected on-chip RAM for deterministic entry.

Each vector entry contains a 32-bit handler address or an implementation-approved compact branch stub.

Sixteen architectural priority levels remain fixed-priority in hardware; multiple sources may route to each level.

# 4. TCM and SRAM Profiles

| Profile | TCM | General SRAM | DMA | Intended use |
| --- | --- | --- | --- | --- |
| Aurora-L | No dedicated TCM required | Banked on-chip SRAM | Simple DMA optional | MCU-oriented control plus occasional DSP |
| Aurora-M | 32-64 KiB ITCM + 32-64 KiB DTCM | Moderate banked SRAM | DMA standard | Target low/mid fixed-point DSP + MCU consolidation |
| Aurora-P | 64-256 KiB ITCM + 64-256 KiB DTCM | Larger SRAM and external DRAM | Multi-channel DMA standard | Sustained streaming and higher throughput |

The fixed ITCM and DTCM windows provide software portability. An implementation advertises actual size through ARCH_VERSION/feature discovery. Access beyond implemented capacity faults rather than silently wrapping. Aurora-L software may place all code and data in general SRAM or flash.

# 5. Memory Attributes and Ordering

Normal memory may permit instruction fetch, speculative prefetch and burst access according to profile.

Device/MMIO memory is strongly ordered. Loads and stores do not reorder across MMIO accesses.

Stores remain architecturally ordered in v1.2.

The four-entry pairing window may move non-faulting register operations around memory instructions only when precise state is preserved.

Loads do not pass older stores unless a future profile explicitly adds address-disambiguation support.

Instruction writes to executable memory require an explicit synchronization operation before execution.

DMA completion does not imply CPU visibility until required memory barriers and ownership rules are satisfied.

# 6. Core-Local Privileged Registers

| Address | Register | Access | Purpose |
| --- | --- | --- | --- |
| 0xF000_0000 | CORE_ID | RO | Implementation and profile identifier |
| 0xF000_0004 | ARCH_VERSION | RO | Architectural version and feature bitmap |
| 0xF000_0010 | PSR | RW privileged fields | Flags, mode, interrupt state, DSP sticky status |
| 0xF000_0014 | VECTOR_BASE | RW privileged | Aligned base address of active vector table |
| 0xF000_0018 | EXC_CAUSE | RO | Current or most recent exception cause |
| 0xF000_001C | EXC_ADDR | RO | Faulting address or restart PC |
| 0xF000_0020 | CYCLE_COUNT | RO | 32-bit free-running coarse monotonic timer |
| 0xF000_0024 | TIMER_COUNT | RW privileged | Single 32-bit coarse countdown timer |
| 0xF000_0028 | TIMER_VECTOR | RW privileged | Interrupt vector pended when timer expires |
| 0xF000_002C | TIMER_CTRL | RW privileged | Enable, hold, pending-clear, tick prescale |
| 0xF000_0030 | STREAM_MASK | RW | One-cycle Q0/Q1 enable mask |
| 0xF000_0034 | DSP_DIRTY | RW privileged | Accumulator/stream extended-state dirty bits |
| 0xF000_0040 | ASC_ENTRY_BASE | RO/RW profile | ASC service entry table location |
| 0xF000_0050 | MPU_CTRL | RW privileged | Memory protection enable and default policy |
| 0xF000_0060 | DMA_GLOBAL_CTRL | RW privileged | Global DMA enable, fault and arbitration policy |

# 7. Countdown Timer and Monotonic Counter

Aurora exposes one 32-bit free-running coarse counter and one 32-bit OS-scheduled countdown timer. The countdown represents only the next scheduled event; privileged software multiplexes all logical timers in a software priority queue.

CYCLE_COUNT increments in coarse timer ticks and wraps modulo 2^32.

TIMER_COUNT decrements in the same or an implementation-advertised coarse tick domain.

A timer tick may represent 2^P CPU cycles; software or ASC translates requested delays.

TIMER_VECTOR selects the interrupt vector pended at expiration.

Expiration-to-pending latency is fixed and documented for each implementation.

The counter implementation may be ripple, segmented, serial or otherwise low-cost and must remain off the CPU critical path.

The baseline timer is one-shot. Privileged software reloads it for periodic scheduling.

# 8. Interrupt Controller MMIO

| Address | Register | Access | Purpose |
| --- | --- | --- | --- |
| 0x1000_0000 | INT_PENDING | RO/W1C | Pending interrupt-source bitmap or bank selector |
| 0x1000_0004 | INT_ENABLE | RW | Interrupt enable bitmap or bank selector |
| 0x1000_0008 | INT_ACTIVE | RO | Currently active source/vector |
| 0x1000_000C | INT_EOI | WO | End-of-interrupt acknowledgement |
| 0x1000_0010 | INT_THRESHOLD | RW privileged | Current minimum accepted priority |
| 0x1000_0040-0x1000_007C | INT_ROUTE[n] | RW privileged | Source-to-vector and subpriority routing |

Routing entries bind a physical source to one of sixteen architectural vectors/priority levels and may also define a deterministic fixed subpriority. EDF or other task scheduling remains software policy.

# 9. DMA MMIO

| Address | Register | Access | Purpose |
| --- | --- | --- | --- |
| 0x1001_0000 | DMA_CTRL | RW | Global DMA control |
| 0x1001_0010 + n*0x40 | DMA_SRC[n] | RW | Channel source address |
| 0x1001_0014 + n*0x40 | DMA_DST[n] | RW | Channel destination address |
| 0x1001_0018 + n*0x40 | DMA_COUNT[n] | RW | Transfer count |
| 0x1001_001C + n*0x40 | DMA_STRIDE[n] | RW | Source/destination stride |
| 0x1001_0020 + n*0x40 | DMA_CFG[n] | RW | Width, burst, circular, interrupt, trigger |
| 0x1001_0024 + n*0x40 | DMA_STATUS[n] | RO/W1C | Busy, complete and fault state |

The number of channels is implementation-defined. Channel zero should exist in DSP-oriented profiles. DMA may transfer among TCM, on-chip SRAM, peripherals and external memory. Writes to ITCM require instruction synchronization before execution.

# 10. Stream and TCM Interaction

Q0 and Q1 use ordinary physical addresses; no separate stream-only address space exists.

DTCM and banked SRAM are preferred low-latency stream sources and sinks.

DMA may double-buffer stream regions while the core operates on the alternate buffer.

Circular addressing may be used in TCM, SRAM or external memory when alignment and region-size rules are met.

The system fabric arbitrates CPU, stream and DMA requests deterministically according to profile.

# 11. External Memory Interface

External-memory targets use a ready/acknowledge request-response interface. Slow devices may insert wait states without changing ISA semantics.

Address, read/write data, byte enables, valid, ready and fault/error are the minimum logical signals.

External SRAM, PSRAM, NOR flash, QSPI/OSPI flash and DRAM controllers may be attached.

Aurora-L need not implement external DRAM.

MMIO and external-memory controllers are fabric slaves; general SRAM is not physically placed in front of them.

# 12. Protection and Privilege

Boot/ASC ROM is executable and privileged-write-protected.

System-control and debug regions are privileged.

User mode may access permitted normal memory and selected peripheral windows.

The MPU defines execute, read, write, device and privilege attributes.

ASC may emulate optional unaligned accesses or unsupported operations, but normal fast-path protection checks remain hardware.

# 13. Discovery and Portability

Software must discover profile features rather than infer them from product names. CORE_ID and ARCH_VERSION report TCM sizes, SRAM banks, DMA channels, external-memory support, timer prescale and optional features.

The canonical windows remain fixed even when a region is absent.

Absent optional regions raise a precise access fault.

Boot firmware publishes a platform descriptor for board-specific peripherals and clocks.

The architecture manual defines addresses; the SoC manual defines peripheral-specific blocks inside MMIO.

# 14. Freeze Checklist

PASS - 32-bit unified physical address space defined.

PASS - Boot/ASC ROM and reset vector fixed.

PASS - Optional ITCM/DTCM windows defined.

PASS - On-chip SRAM, MMIO and external-memory windows defined.

PASS - Core-local system-register window defined.

PASS - Single countdown timer and coarse monotonic counter defined.

PASS - Interrupt-controller and DMA base windows defined.

PASS - Ordering, protection and discovery rules defined.