Aurora v1.2 Cycle-Accurate Simulator Specification

AUR-SIM-001 | Canonical Draft 1

# 1. Simulator Modes

Functional mode: architectural correctness without cycle timing

Cycle-accurate mode: pipeline, window, scoreboard, memory and interrupt timing

Trace-replay mode: deterministic reproduction of failures

# 2. State Model

GPRs, PSR, A0/A1, Q0/Q1, PC/SP/LR

Four-entry issue window and per-entry status

Scoreboard and outstanding completion records

Memory map, TCM/SRAM timing, MMIO and DMA state

Interrupt pending/active state and countdown timer

# 3. Per-Cycle Order

Apply external events and memory responses

Retire oldest completed instructions

Recognize precise interrupts

Advance multi-cycle units

Execute/launch selected issue pair

Select and compact issue window

Fetch and decode new words

# 4. Trace Format

Cycle number, PC, decoded instruction, issue lane, resource mask

Register/flag/accumulator writes

Memory requests/responses and stream consumption

Window contents, scoreboard stalls and branch recovery

Interrupt and exception events

# 5. Validation

The simulator must consume the canonical ISA JSON and compare against directed tests, random programs, and RTL traces.