# Aurora Architecture v1.2

Complete Instruction Reference and Baseline Timing Table

Canonical ISA Draft 1 - August 2026

This document defines the programmer-visible behavior, flags, exceptions, issue resources, baseline latency, initiation interval, mode interactions, and examples for the Aurora v1.2 instruction families. Exact binary encodings are defined in the companion Instruction Formats and Opcode Map specification.

# 1. Timing and Issue Conventions

Latency is the number of core cycles from issue until the architectural result is available for a dependent instruction through forwarding or writeback.

Initiation interval (II) is the minimum cycles between issuing operations to the same functional resource when operands and destinations are independent.

Slot A favors scalar ALU, address, control, compare, branch, and simple register-layout work. Slot B favors multiply, divide, packed-wide DSP, reduction, stream, and memory operations.

The four-entry pairing window may pair the oldest instruction with one independent younger non-barrier instruction. It does not change architectural order.

Memory latency assumes ITCM/DTCM or single-cycle on-chip SRAM. External-memory latency is implementation- and target-dependent and is scoreboarded.

Multi-cycle instructions have bounded baseline timing but may complete later when waiting on memory, streams, or shared resources.

## 1.1 Flag notation

Z = zero    N = negative/sign    C = carry/borrow/last-shifted bit    V = signed overflow
Q = sticky DSP status where defined    - = unchanged

## 1.2 Mode notation

S32 = scalar 32-bit   P16 = packed 2 x 16 per register / 4 x 16 per pair
P8 = packed 4 x 8 per register / 8 x 8 per pair   L16/L8 = low-lane masked
P64 = paired 64-bit scalar emulation

# 2. Consolidated Baseline Timing Table

| Family | Slot | Latency | II | Comment |
| --- | --- | --- | --- | --- |
| ADD/SUB/ADC/SBC | A | 1 | 1 | All ALU modes |
| AND/OR/XOR/ANDN | A | 1 | 1 | All ALU modes |
| CMP/TST | A | 1 | 1 | No destination |
| LSL/LSR/ASR/ROL/ROR S32/P8/P16 | A | 1 | 1 | Lane-local |
| LSL/LSR/ASR/ROL/ROR P64 | A/B | 2-3 | 1 | Scoreboarded pair |
| MUL 8/16 packed | B | 2-4 | 1-2 | Full products |
| MUL 32 x 32 | B | 4-8 | 2 | Pair result |
| DIV/DIVSTEP | B | variable bounded | 1 | Multi-cycle |
| DOT/SAD | B | 3-5 | 1-2 | Reduction pipeline |
| REDUCE.ADD/logic | B | 2-3 | 1 | A0/A1 result |
| REDUCE.MIN/MAX/ABSMAX | B | 2-4 | 1 | Comparator tree |
| PACK/UNPACK/ZIP/UNZIP/BSWAP/WSWAP | A | 1 | 1 | Strong pass-through candidate |
| LD/ST scalar TCM | B | 1-2 | 1 | Memory scoreboard |
| LD.P/ST.P TCM | B | 2 | 1-2 | Atomic pair |
| Branch predicted correctly | A | 1 | 1 | Backward-taken |
| Branch mispredict | A | 2-3 | 1 | Flush/refill |
| CALL/RET | A | 1-2 | 1 | R13 link |
| PUSH/POP scalar | B | 1-2 | 1 | R14 implicit |
| QPEEK | B | 1 | 1 | No consume |
| QPOP/QPUSH | B | 1-2 | 1 | May wait on queue |
| SETMODE/QMASK | A | 1 | 1 | State barrier |
| MRS/MSR | A | 1-2 | 1 | Special register |
| ASC entry | A | implementation | - | Privileged transfer |

# 3. Scalar and Packed ALU Instructions

| Mnemonic | Operands | Semantics | Flags | Slot | Latency | II | Faults | Mode | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADD | Rd,Ra,Rb | Rd=Ra+Rb | ZNCV | A | 1 | 1 | - | S32/P16/P8/L16/L8/P64* | P64 is multi-cycle pair form |
| ADC | Rd,Ra,Rb | Rd=Ra+Rb+C | ZNCV | A | 1 | 1 | - | S32/L16/L8/P64* | Used for multiword arithmetic |
| SUB | Rd,Ra,Rb | Rd=Ra-Rb | ZNCV | A | 1 | 1 | - | All ALU | Borrow convention defined by C |
| SBC | Rd,Ra,Rb | Rd=Ra-Rb-(1-C) | ZNCV | A | 1 | 1 | - | S32/L16/L8/P64* | Multiword subtract |
| AND | Rd,Ra,Rb | Bitwise/lane AND | ZN-- | A | 1 | 1 | - | All ALU | L8/L16 preserve upper bits |
| OR | Rd,Ra,Rb | Bitwise/lane OR | ZN-- | A | 1 | 1 | - | All ALU |  |
| XOR | Rd,Ra,Rb | Bitwise/lane XOR | ZN-- | A | 1 | 1 | - | All ALU |  |
| ANDN | Rd,Ra,Rb | Ra & ~Rb | ZN-- | A | 1 | 1 | - | All ALU | Not a reduction operator |
| CMP | Ra,Rb | Set flags from Ra-Rb | ZNCV | A | 1 | 1 | - | All ALU | No destination |
| TST | Ra,Rb | Set flags from Ra & Rb | ZN-- | A | 1 | 1 | - | All ALU | No destination |
| MIN | Rd,Ra,Rb | Signed/unsigned lane min | ZN-- | A | 1 | 1 | - | S32/P16/P8/L16/L8 | Signedness from mode |
| MAX | Rd,Ra,Rb | Signed/unsigned lane max | ZN-- | A | 1 | 1 | - | S32/P16/P8/L16/L8 | Signedness from mode |

## 3.1 Low-lane write masking

L8:  Rd = (old_Rd & 0xFFFFFF00) | result[7:0]
L16: Rd = (old_Rd & 0xFFFF0000) | result[15:0]

To avoid a third register-file read port, low-lane arithmetic is preferably encoded as a two-address operation in which Rd is also source A. The scoreboard remains register-granular.

# 4. Immediate, Shift, and Rotate Instructions

| Mnemonic | Operands | Semantics | Flags | Slot | Latency | II | Faults | Mode | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADDI | Rd,Ra,#imm | Rd=Ra+sext(imm) | ZNCV | A | 1 | 1 | - | S32/L16/L8 | Extension word for long immediate |
| ANDI/ORI/XORI | Rd,Ra,#imm | Logic with immediate | ZN-- | A | 1 | 1 | - | S32/L16/L8 |  |
| MOVI | Rd,#imm | Load immediate | ZN-- | A | 1 | 1 | - | S32 | Long form uses 0xE escape |
| LSL | Rd,Ra,#n | Logical left shift | ZNC- | A | 1 | 1 | - | All | P64: 2-3 cycles |
| LSR | Rd,Ra,#n | Logical right shift | ZNC- | A | 1 | 1 | - | All |  |
| ASR | Rd,Ra,#n | Arithmetic right shift | ZNC- | A | 1 | 1 | - | S32/P16/P8/L16/L8/P64 | P64 sign is bit 63 |
| ROL | Rd,Ra,#n | Rotate left | ZNC- | A | 1 | 1 | - | All | Count modulo active width |
| ROR | Rd,Ra,#n | Rotate right | ZNC- | A | 1 | 1 | - | All | Count modulo active width |
| SHR.R | Rd,Ra,#n | Rounded arithmetic right shift | ZNCQ | A/B | 1-3 | 1 | - | S32/L16/L8/P64 | Optional profile if encoding retained |

In paired-64 mode, even register fields name 64-bit pairs, while SP and PC may be selected as zero-extended source operands. Pair shifts reuse 32-bit hardware and are intentionally multi-cycle.

# 5. Multiply and Divide Instructions

| Mnemonic | Operands | Semantics | Flags | Slot | Latency | II | Faults | Mode | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MUL | Rd,Ra,Rb | Full-width signed/unsigned product | ZN-Q | B | 2-8 | 1-2 | Illegal pair | P8/P16/S32/P64 | Mode selects lane/product width |
| MULH | Rd,Ra,Rb | High half of scalar product | ZN-Q | B | 3-6 | 2 | - | S32 | If retained by final opcode map |
| MAC | A#,Ra,Rb | A=A+product/reduced products | ---Q | B | 3-6 | 1-2 | - | P8/P16/S32 | Accumulator select bit |
| MAS | A#,Ra,Rb | A=A-product | ---Q | B | 3-6 | 1-2 | - | P8/P16/S32 | Separate from REDUCE fold model |
| DIV | Rd,Ra,Rb | Integer quotient | ZNVQ | B | bounded multi | 1 | Divide by zero | S32 | No scaled-divide opcode |
| DIVSTEP | state,operand | One restoring/nonrestoring step | ZN-Q | B | 1 | 1 | - | S32 | Compiler/runtime building block |

# 6. DSP Reduction, DOT, and SAD Instructions

| Mnemonic | Operands | Semantics | Flags | Slot | Latency | II | Faults | Mode | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DOT | A#,Ra,Rb | A=sum(lane products) | ---Q | B | 3-5 | 1-2 | Stream underflow | P8/P16 | Overwrite or fold |
| SAD | A#,Ra,Rb | A=sum(abs(Ra_i-Rb_i)) | ---Q | B | 3-5 | 1-2 | Stream underflow | P8/P16 | Overwrite or fold |
| RED.ADD | A#,Ra | A=sum(lanes) | ---Q | B | 2-3 | 1 | Stream underflow | P8/P16 | Fold uses + |
| RED.AND | A#,Ra | A=AND(lanes) | ---Q | B | 2 | 1 | Stream underflow | P8/P16 | Fold uses AND |
| RED.OR | A#,Ra | A=OR(lanes) | ---Q | B | 2 | 1 | Stream underflow | P8/P16 | Fold uses OR |
| RED.XOR | A#,Ra | A=XOR(lanes) | ---Q | B | 2 | 1 | Stream underflow | P8/P16 | Fold uses XOR |
| RED.MIN | A#,Ra | A=min(lanes) | ---Q | B | 2-4 | 1 | Stream underflow | P8/P16 | Fold uses min(A,x) |
| RED.MAX | A#,Ra | A=max(lanes) | ---Q | B | 2-4 | 1 | Stream underflow | P8/P16 | Fold uses max(A,x) |
| RED.ABSMAX | A#,Ra | A=max(abs(lanes)) | ---Q | B | 3-4 | 1 | Min-negative policy | P8/P16 | Fold uses max |
| RED.COUNT | A#,Ra | A=count(true lanes) | ---Q | B | 2-3 | 1 | Stream underflow | P8/P16 | Fold uses addition |

## 6.1 Overwrite and fold semantics

overwrite: A := reduce(current chunk)
fold:      A := combine(A, reduce(current chunk))

The combine operator is specific to the reduction. This permits unbounded stream reductions across repeated four-lane or eight-lane chunks without materializing intermediate scalars.

# 7. Register Layout and Shuffle Instructions

| Mnemonic | Operands | Semantics | Flags | Slot | Latency | II | Faults | Mode | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | Rd,Rs | Full 32-bit copy | ---- | A | 1 | 1 | - | All | Ignores stream interception and L8/L16 mask |
| BSWAP | Rd,Rs | Reverse four bytes | ZN-- | A | 1 | 1 | - | All | Strong pairing candidate |
| WSWAP | Rd,Rs | Swap low/high 16-bit halves | ZN-- | A | 1 | 1 | - | All |  |
| PACK | Rd,Ra,Rb | Pack selected low lanes | ZN-Q | A | 1 | 1 | - | All | Register-only |
| UNPACK | Rd,Rs | Widen selected lanes | ZN-Q | A | 1 | 1 | - | All | Register-only |
| ZIP | Rd,Ra,Rb | Interleave lanes | ZN-- | A | 1 | 1 | - | P8/P16 |  |
| UNZIP | Rd,Rs | Deinterleave lanes | ZN-- | A | 1 | 1 | - | P8/P16 |  |
| SHUF | Rd,Rs,#pattern | Byte/word permutation | ZN-- | A | 1-2 | 1 | Illegal pattern | P8/P16/S32 | Extended pattern if needed |

These operations are unaffected by low-lane write masking so software can move upper stored bytes or halfwords into lane zero. They are non-faulting, register-only operations and are preferred pass-through candidates in the four-entry issue window.

# 8. Memory Instructions

| Mnemonic | Operands | Semantics | Flags | Slot | Latency | II | Faults | Mode | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LD.B/LD.H/LD.W | Rd,[Ra+off] | Load and extend | ZN-- | B | 1-2+ | 1 | Align/bus/protection | S32 | Signedness by subfunction |
| ST.B/ST.H/ST.W | Rs,[Ra+off] | Store low 8/16/32 bits | ---- | B | 1-2+ | 1 | Align/bus/protection | S32 | Stores remain ordered |
| LD.P | Re,[Ra+off] | Atomic 64-bit pair load | ZN-- | B | 2+ | 1-2 | Align/bus/protection | P64 | Low word at lower address |
| ST.P | Re,[Ra+off] | Atomic 64-bit pair store | ---- | B | 2+ | 1-2 | Align/bus/protection | P64 | R12:R13 conditional |
| LDA | Rd,[Ra+off] | Address calculation only | ZN-- | A | 1 | 1 | - | S32/P64 | No memory access |

Loads may pass unrelated register-only work but do not pass older stores in the v1.2 baseline.

MMIO accesses are ordering barriers and never reordered.

TCM accesses are deterministic. General SRAM and external-memory accesses use ready/ack and remain scoreboarded.

Unaligned accesses may trap or be handled by ASC according to implementation profile.

# 9. Branch, Call, Loop, and Stack Instructions

| Mnemonic | Operands | Semantics | Flags | Slot | Latency | II | Faults | Mode | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B.cc | target | Conditional PC-relative branch | ---- | A | 1 / 2-3 | 1 | Target/protection | S32 | Backward branches predicted taken |
| JMP | target/Rs | Unconditional transfer | ---- | A | 1-2 | 1 | Target/protection | S32 |  |
| CALL | target/Rs | R13=return PC; PC=target | ---- | A | 1-2 | 1 | Target/protection | S32 | Clobbers LR |
| RET | R13 | PC=R13 | ---- | A | 1-2 | 1 | Target/protection | S32 | May be branch alias |
| LOOPSET | count,target | Configure low-overhead loop | ---- | A | 1 | 1 | Privilege/profile | S32 | Exact state in loop/AGU class |
| PUSH | Rs/list | SP-=size; store | ---- | B | 1-2+ | 1 | Stack/bus | S32/P64 | R14 implicit |
| POP | Rd/list | load; SP+=size | ---- | B | 1-2+ | 1 | Stack/bus | S32/P64 |  |

# 10. Stream and Queue Instructions

| Mnemonic | Operands | Semantics | Flags | Slot | Latency | II | Faults | Mode | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QMASK | #mask | Enable default Q0/Q1 interception | ---- | A | 1 | 1 | - | All | State barrier |
| QPEEK | Rd,Q# | Read current element, no advance | ZN-- | B | 1 | 1 | Empty queue | P8/P16/S32/P64 |  |
| QPOP | Rd,Q# | Consume current element | ZN-- | B | 1-2+ | 1 | Empty queue | P8/P16/S32/P64 | Retirement advances state |
| QPUSH | Q#,Rs | Produce element | ---- | B | 1-2+ | 1 | Full queue | P8/P16/S32/P64 |  |
| QDUP | Q# | Duplicate local/LIFO top | ---- | B | 1 | 1 | Invalid top/full | All |  |
| QREPL | Q#,Rs | Replace top/head | ---- | B | 1 | 1 | Invalid top | All | Occupancy unchanged |
| QCFG | Q#,fields | Configure address/stride/count/width | ---- | A/B | 1-3 | 1 | Privilege/config | All | Extension words likely |
| QON/QOFF | Q# | Enable/disable stream engine | ---- | A | 1 | 1 | - | All | One-cycle state change |

Stream-capable DOT, SAD, REDUCE, and memory forms contain a consume/local override. Register-only layout instructions never consume streams. Address, count, and circular-wrap state advance only after successful retirement.

# 11. System, Timer, Interrupt, and ASC Instructions

| Mnemonic | Operands | Semantics | Flags | Slot | Latency | II | Faults | Mode | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MRS | Rd,SReg | Read special register | ZN-- | A | 1-2 | 1 | Privilege | S32 | Includes coarse TIME_COUNT |
| MSR | SReg,Rs | Write special register | ---- | A | 1-2 | 1 | Privilege | S32 | State barrier |
| SETMODE | #mode | Set arithmetic interpretation | ---- | A | 1 | 1 | Illegal mode | All | Public ABI restores S32 |
| TIMERLD | Rs/vector | Atomically load countdown and vector | ---- | A/B | 1-2 | 1 | Privilege | S32 | Countdown in coarse ticks |
| IRET | - | Restore interrupt PC/PSR | all | A | 2+ | - | Privilege/frame | S32 | Ordered privileged return |
| EI/DI | - | Enable/disable interrupt recognition | ---- | A | 1 | 1 | Privilege | S32 |  |
| BARRIER | class | Order memory/stream/system effects | ---- | A | 1+ | - | - | S32 | Issue-window barrier |
| ASC | service | Enter Architecture Service Code | ---- | A | impl. | - | Privilege/service | S32 | Protected ROM/ITCM transfer |

## 11.1 Timer behavior

TIME_COUNT is a 32-bit free-running counter of coarse timer ticks and wraps modulo 2^32.

TIMER_COUNT is a single 32-bit one-shot countdown. Privileged software multiplexes all logical timers onto the next expiration.

The implementation may use ripple, segmented, serial, or other low-cost counter circuitry behind a prescaler.

Expiration-to-interrupt posting latency is fixed and documented; cycle-exact scheduling is not required.

# 12. Exceptions, Ordering, and Precise State

Illegal opcode, illegal mode, odd or prohibited pair destination, divide by zero, alignment, bus, memory-protection, privilege, stream underflow/overflow, and ASC service faults are precise.

Potentially faulting operations may execute for multiple cycles but cannot allow younger architectural state to retire past them.

Simple non-faulting unary/register operations may bypass older stalled instructions when dependency and barrier checks pass.

Stores, MMIO, stream configuration, mode changes, privileged operations, and unresolved control transfers are ordering barriers.

The implementation may maintain small per-window completion records; this is not a general speculative reorder buffer.

# 13. Compiler Scheduling Model

The compiler is the primary scheduler; hardware dynamic pairing repairs short local gaps within four decoded instructions.

Alternate A0/A1 to hide accumulator feedback when kernel semantics permit.

Pair scalar address/control work in Slot A with DSP, stream, multiply, reduction, or memory work in Slot B.

Prefer BSWAP, WSWAP, PACK, UNPACK, ZIP, UNZIP, MOV, compare, and simple shifts as pass-through fillers.

Model Q0/Q1 consumption, A0/A1 access, pair aliases, LR liveness, memory ports, writeback paths, and functional-unit initiation intervals.

Use L8/L16 upper-lane storage conservatively; hazards remain whole-register granular.

Use pair loads/stores for 64-bit spills and do not assume extra pair-only scratch registers.

# 14. Worked Scheduling Examples

## 14.1 Streaming DOT with dual accumulators

DOT       A0, Q0, Q1       ; initialize first chunk
    ADDI      R6, R6, #8       ; Slot A work can pair
    DOT.ACC   A1, Q0, Q1       ; alternate feedback chain
    CMP       R6, R7
    DOT.ACC   A0, Q0, Q1
    B.LT      loop

## 14.2 Low-byte scalar storage

; R4 contains four packed byte temporaries
    BSWAP     R4, R4           ; move a selected upper byte toward lane zero
    ADD.L8    R4, R5           ; update low byte, preserve upper 24 bits
    BSWAP     R4, R4           ; restore layout

## 14.3 Paired-64 fixed-point extraction

MUL       R2:R3, R4, R6    ; full product
    ASR.P64   R2:R3, #15       ; multi-cycle paired shift
    MOV       R0, R2           ; scaled low 32-bit result

## 14.4 Four-entry dynamic pairing

Window: [ DOT A0,Q0,Q1 | DOT.ACC A0,Q0,Q1 | WSWAP R6 | ADDI R7,#1 ]
Issue:   oldest DOT in Slot B + independent WSWAP in Slot A
Remain:  [ DOT.ACC A0,Q0,Q1 | ADDI R7,#1 ] in original order

# 15. Remaining Freeze Items

| Item | Status | Action |
| --- | --- | --- |
| Exact flag convention for subtraction C/borrow | Open | Must be uniform across SUB, SBC, CMP. |
| Saturation and rounding subset | Open | Current baseline avoids broad saturation; optional rounded shift requires final decision. |
| Exact multiply and divide latency by implementation profile | Profile | Architecture provides bounded classes; RTL synthesis sets numbers. |
| Exact stream underflow behavior | Open | Trap, stall, or programmable behavior must be finalized per queue mode. |
| Exact branch displacement and extension rules | Encoding | Defined in opcode map but requires final bit-level audit. |
| Instruction-by-instruction binary encodings | Next | Generate machine-readable ISA database and assembler tables. |
| Silicon-validated timing | Next | Replace provisional latency values after cycle-accurate RTL and synthesis. |