Aurora Instruction Set Reference

Phase 1 Baseline, Revision 1.0

# 1. Encoding model

Every base instruction is 16 bits. Bits 15:12 select one of sixteen primary families. R0-R15 use direct four-bit fields. Selected instructions consume one 16-bit extension word. A prefix and its consumer form one precise restartable instruction group.

| Primary | Family | Default format |
| --- | --- | --- |
| 0 | System/control | class[11:8], function[7:0] |
| 1 | Move/immediate | Rd[11:8], imm8[7:0] |
| 2 | Load | Rd, Rb, mode/offset |
| 3 | Store | Rs, Rb, mode/offset |
| 4 | Scalar arithmetic | Rd, Ra, Rb / subfn |
| 5 | Logic/compare/bit | Rd, Ra, Rb / subfn |
| 6 | Shift/rotate | Rd, Rs, subfn/amount |
| 7 | Scalar multiply/divide | Rd/pair, Ra, Rb / subfn |
| 8 | Packed/SIMD | Rd/pair, Ra, Rb / subfn |
| 9 | Pack/unpack/convert | Rd, Rs, subfn |
| A | Reduction | Rd/pair, Rs/Ra, subfn |
| B | Stream/stack/DMA | class, operands/function |
| C | Conditional branch | cond[11:8], disp8[7:0] |
| D | Jump/call/return | class[11:8], operand[7:0] |
| E | Extended DSP | Rd/pair, operands/subfn |
| F | Prefix/extensions | class[11:8], payload[7:0] |

# 2. Exact primary and sub-opcode allocation

## 0x0 System/control

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 00 | NOP | No operation |
| 01 | WAIT | Wait for interrupt/event |
| 02 | HALT | Halt until reset or permitted wake |
| 03 | EI | Enable maskable interrupts |
| 04 | DI | Disable maskable interrupts |
| 05 | SYNC | Order selected scalar/stream/DMA accesses |
| 06 | ISYNC | Synchronize instruction fetch |
| 07 | BREAK | Debug breakpoint |
| 08 | CLRQ | Clear sticky Q |
| 09 | RPSR | Read PSR to Rd encoded in extension |
| 0A | WPSR | Write permitted PSR fields from Rs |
| 0B | TRAP | Software exception; vector in extension |
| 0C | RETI | Return from interrupt |
| 0D | SVC | Supervisor call |
| 0E | FENCEIO | Strong device-ordering fence |
| 0F-FF | RES | Reserved |

## 0x1 Move/immediate

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| Rd:imm8 | MOVI8 | Sign-extended 8-bit immediate |
| sub0 | MOV | Rd <- Rs |
| sub1 | MOVU8 | Zero-extended imm8 |
| sub2 | LUI16 | Load upper 16 via extension |
| sub3 | MOV16 | Load 16-bit immediate via extension |
| sub4 | MOV32 | Load 32-bit immediate via two extension words |
| sub5 | CMOVcc | Conditional move; condition in extension |
| sub6-F | RES | Reserved |

## 0x2 Load

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | LDBU | Unsigned byte load |
| 1 | LDBS | Signed byte load |
| 2 | LDHU | Unsigned halfword load |
| 3 | LDHS | Signed halfword load |
| 4 | LDW | 32-bit word load |
| 5 | LDD | 64-bit pair load |
| 6 | LDW.POST | Word load, post-increment base |
| 7 | LDW.PRE | Pre-decrement base, word load |
| 8 | LDX | Indexed/extended load |
| 9-F | RES | Reserved |

## 0x3 Store

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | STB | Byte store |
| 1 | STH | Halfword store |
| 2 | STW | 32-bit word store |
| 3 | STD | 64-bit pair store |
| 4 | STW.POST | Word store, post-increment base |
| 5 | STW.PRE | Pre-decrement base, word store |
| 6 | STX | Indexed/extended store |
| 7-F | RES | Reserved |

## 0x4 Scalar arithmetic

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | ADD | Rd=Ra+Rb |
| 1 | ADDS | ADD and update NZCV |
| 2 | ADC | Add with carry |
| 3 | ADCS | ADC and update NZCV |
| 4 | SUB | Rd=Ra-Rb |
| 5 | SUBS | SUB and update NZCV |
| 6 | SBC | Subtract with borrow/no-borrow convention |
| 7 | SBCS | SBC and update NZCV |
| 8 | MIN | Signed min |
| 9 | MAX | Signed max |
| A | UMIN | Unsigned min |
| B | UMAX | Unsigned max |
| C | ABS | Scalar absolute value |
| D | NEG | Two's-complement negate |
| E | ADDI4 | Signed short immediate add |
| F | SUBI4 | Signed short immediate subtract |

## 0x5 Logic/compare/bit

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | AND | Bitwise AND |
| 1 | OR | Bitwise OR |
| 2 | XOR | Bitwise XOR |
| 3 | BIC | Ra AND NOT Rb |
| 4 | NOT | Bitwise complement |
| 5 | CMP | Ra-Rb; flags only |
| 6 | CMPI4 | Compare signed short immediate |
| 7 | TST | Ra AND Rb; flags only |
| 8 | CLZ | Count leading zeros |
| 9 | CTZ | Count trailing zeros |
| A | POPCNT | Population count |
| B | BITREV | Reverse 32 bits |
| C | BEXT | Bit-field extract; extension defines field |
| D | BINS | Bit-field insert; extension defines field |
| E-F | RES | Reserved |

## 0x6 Shift/rotate

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | LSL.I | Logical left, immediate |
| 1 | LSR.I | Logical right, immediate |
| 2 | ASR.I | Arithmetic right, immediate |
| 3 | LSRR.I | Rounded logical right, immediate |
| 4 | ASRR.I | Rounded arithmetic right, immediate |
| 5 | ROL.I | Rotate left, immediate |
| 6 | ROR.I | Rotate right, immediate |
| 7 | RCL.I | Rotate through carry left |
| 8 | RCR.I | Rotate through carry right |
| 9 | LSL.R | Register shift amount |
| A | LSR.R | Register shift amount |
| B | ASR.R | Register shift amount |
| C | ROR.R | Register rotate amount |
| D-F | RES | Reserved |

## 0x7 Scalar multiply/divide

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | MULLO | Low 32 bits unsigned/signed-neutral |
| 1 | MULH | High 32 bits signed |
| 2 | UMULH | High 32 bits unsigned |
| 3 | MUL64 | Signed 32x32 -> 64 pair |
| 4 | UMUL64 | Unsigned 32x32 -> 64 pair |
| 5 | DIV | Signed quotient |
| 6 | UDIV | Unsigned quotient |
| 7 | DIVREM | Signed quotient/remainder pair |
| 8 | UDIVREM | Unsigned quotient/remainder pair |
| 9-F | RES | Reserved |

## 0x8 Packed/SIMD

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | PADD16 | 2x16 wrapping add |
| 1 | PSUB16 | 2x16 wrapping subtract |
| 2 | PADD8 | 4x8 wrapping add |
| 3 | PSUB8 | 4x8 wrapping subtract |
| 4 | QADD16 | 2x16 signed saturating add |
| 5 | QSUB16 | 2x16 signed saturating subtract |
| 6 | QADD8 | 4x8 signed saturating add |
| 7 | QSUB8 | 4x8 signed saturating subtract |
| 8 | PMIN16 | 2x16 signed min |
| 9 | PMAX16 | 2x16 signed max |
| A | PMIN8 | 4x8 signed min |
| B | PMAX8 | 4x8 signed max |
| C | PABSD16 | 2x16 absolute difference |
| D | PABSD8 | 4x8 absolute difference |
| E | PCMP16 | 2x16 comparison mask; condition extension |
| F | PCMP8 | 4x8 comparison mask; condition extension |

## 0x9 Pack/unpack/convert

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | PACK16 | Truncate two 32-bit lanes to 2x16 |
| 1 | PACK8 | Truncate four 16-bit lanes to 4x8 |
| 2 | QPACK16 | Signed saturating 32->16 |
| 3 | QPACKU16 | Unsigned saturating 32->16 |
| 4 | QPACK8 | Signed saturating 16->8 |
| 5 | QPACKU8 | Unsigned saturating 16->8 |
| 6 | UNPACK16L | Widen low 16 lane |
| 7 | UNPACK16H | Widen high 16 lane |
| 8 | UNPACK8L | Widen low two 8-bit lanes |
| 9 | UNPACK8H | Widen high two 8-bit lanes |
| A | ZIP16 | Interleave 16-bit lanes |
| B | UNZIP16 | Deinterleave 16-bit lanes |
| C | ZIP8 | Interleave 8-bit lanes |
| D | UNZIP8 | Deinterleave 8-bit lanes |
| E-F | RES | Reserved |

## 0xA Horizontal/reduction

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | HADD16 | Sum 2x16 to 32-bit scalar |
| 1 | HADD8 | Sum 4x8 to 32-bit scalar |
| 2 | HMIN16 | Horizontal signed min |
| 3 | HMAX16 | Horizontal signed max |
| 4 | HMIN8 | Horizontal signed min |
| 5 | HMAX8 | Horizontal signed max |
| 6 | HADDW16 | Sum two widened 32-bit lanes -> 64 pair |
| 7 | HADDW8 | Sum four widened 16-bit lanes -> 32 |
| 8 | SAD16 | Sum 2x16 absolute differences |
| 9 | SAD8 | Sum 4x8 absolute differences |
| A | HOR | Horizontal OR |
| B | HXOR | Horizontal XOR |
| C | HAND | Horizontal AND |
| D-F | RES | Reserved |

## 0xB Stream/stack/DMA

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 00 | STRCFG0 | Configure R0 stream from descriptor registers |
| 01 | STRCFG4 | Configure R4 stream |
| 02 | STREN0 | Enable R0 stream |
| 03 | STREN4 | Enable R4 stream |
| 04 | STRDIS0 | Disable R0 stream |
| 05 | STRDIS4 | Disable R4 stream |
| 06 | STRSYNC | Synchronize selected streams |
| 07 | STRFLUSH | Flush selected stream writes |
| 08 | LIFOON | Couple R0/R4 LIFO |
| 09 | LIFOOFF | Decouple after synchronization |
| 0A | PUSH | Push one register on R13 stack |
| 0B | POP | Pop one register from R13 stack |
| 0C | PUSHM | Push register mask in extension |
| 0D | POPM | Pop register mask in extension |
| 10 | DMACFG | Configure DMA channel |
| 11 | DMASTART | Start DMA channel |
| 12 | DMASTOP | Stop DMA channel |
| 13 | DMASYNC | Synchronize DMA channel |
| 14-FF | RES | Reserved |

## 0xC Conditional branch

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | AL | Always |
| 1 | EQ | Z=1 |
| 2 | NE | Z=0 |
| 3 | CS/HS | C=1 |
| 4 | CC/LO | C=0 |
| 5 | MI | N=1 |
| 6 | PL | N=0 |
| 7 | VS | V=1 |
| 8 | VC | V=0 |
| 9 | HI | C=1 and Z=0 |
| A | LS | C=0 or Z=1 |
| B | GE | N=V |
| C | LT | N!=V |
| D | GT | Z=0 and N=V |
| E | LE | Z=1 or N!=V |
| F | NV | Never/reserved for patching |

## 0xD Jump/call/return

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | JREL | Signed relative jump; extension |
| 1 | JREG | Jump register |
| 2 | CALLREL | Relative call; extension |
| 3 | CALLREG | Call register |
| 4 | RET | PC <- R14 |
| 5 | RETI | Return from interrupt |
| 6 | TAIL | Tail-call register |
| 7 | SWITCH | Indexed jump-table assist |
| 8-F | RES | Reserved |

## 0xE Extended DSP/arithmetic

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | PMUL16 | 2x16 widening signed multiply -> pair |
| 1 | PUMUL16 | 2x16 widening unsigned multiply -> pair |
| 2 | PMUL8 | 4x8 widening signed multiply -> pair |
| 3 | PUMUL8 | 4x8 widening unsigned multiply -> pair |
| 4 | PMAC16 | 2x16 widening MAC |
| 5 | PMSUB16 | 2x16 widening multiply-subtract |
| 6 | PMAC8 | 4x8 widening MAC |
| 7 | PMSUB8 | 4x8 widening multiply-subtract |
| 8 | DOT16 | 2x16 dot product -> 64 pair |
| 9 | DOT8 | 4x8 dot product -> 32 |
| A | PSHL16 | Packed 2x16 shift |
| B | PSHL8 | Packed 4x8 shift |
| C | PRSHR16 | Packed rounded right shift |
| D | PRSHR8 | Packed rounded right shift |
| E-F | RES | Reserved |

## 0xF Prefix/extensions

| Code/subfunction | Mnemonic | Normative meaning |
| --- | --- | --- |
| 0 | IMMHI | High immediate payload for next instruction |
| 1 | IMM32 | Begins 32-bit immediate group |
| 2 | DISP16 | 16-bit displacement for next memory/control instruction |
| 3 | REGMASK | Register mask for PUSHM/POPM |
| 4 | COND | Condition selector for CMOV/PCMP |
| 5 | STREAMX | Extended stream configuration payload |
| 6 | DMAX | Extended DMA payload |
| 7 | SYSX | Extended privileged/system payload |
| 8-F | RES | Reserved |

# 3. Normative execution rules

All scalar arithmetic is modulo 2^32 unless an explicit saturating operation is selected.

All packed wrapping operations wrap independently within each lane.

Saturating operations clamp each lane and set sticky Q if any lane clamps.

64-bit destinations name an even register; the following odd register is implied. Odd pair bases trap.

Loads, stores, streams, multiply, divide, and reductions may be scoreboarded and complete after the nominal pipeline stage.

Instructions retire in program order. Faulting instructions do not partially update registers, PSR, stream state, or memory.

Branches use signed displacement in 16-bit instruction units relative to the following instruction.

R0 and R4 behave as ordinary registers when their stream mode is disabled.

When stream mode is enabled, an architectural read or write consumes or produces one configured 8-, 16-, or 32-bit element and advances only on successful completion.

No baseline 4-bit stream or packed lane format exists.

# 4. Baseline timing classes

| Class | Latency | Initiation interval | Notes |
| --- | --- | --- | --- |
| Scalar ALU/logic/shift | 1 | 1 | Forwarding may remove adjacent dependency stall |
| Packed ALU/pack/narrow | 1 | 1 | Q updates retire in order |
| Load hit in closely coupled SRAM | 2 | 1 per bank | Longer on bank conflict or wait-state memory |
| Store enqueue | 1 | 1 per bank | Visibility ordered by store/stream rules |
| Packed multiply/MAC | 2 | 1 | Shared compressor resource |
| Scalar multiply | 4 | 2 | Shared compressor resource |
| Reduction/SAD/dot | 2 | 1 | Shared compressor resource |
| Divide | <=34 | non-pipelined | Scoreboarded iterative divider |
| Branch | resolved EX/MEM | - | Mispredict penalty implementation profile: 2 cycles |