Aurora Phase 1 Validation and Architecture Freeze Report

Revision 1.0

# 1. Freeze decision

Phase 1 is APPROVED as an architectural baseline. Approval means that programming-model semantics, instruction families, exact Phase 1 sub-opcode allocation, ABI, exceptions, ordering, streams, memory profile, and baseline timing classes are frozen. It does not constitute RTL sign-off or a guaranteed performance claim.

# 2. Architecture review checklist

| Gate | Status | Evidence / limitation |
| --- | --- | --- |
| Product niche defined | PASS | DSP-first, MCU-capable, low-cost deterministic embedded processor. |
| Representative workloads identified | PASS | FIR, FFT, image/SAD, matrix, PCA/ICA primitives, control, RTOS. |
| Compiler synthesis considered | PASS | No dedicated PCA/ICA/FFT opcodes; general packed, MAC, reduction, stream primitives. |
| Instruction savings plausible | PASS | Streams remove address instructions; MAC/dot/SAD fuse common sequences; dual issue overlaps scalar control. |
| Silicon reuse maximized | PASS | Shared compressor tree; banked commodity SRAM; one iterative divider. |
| Critical-path risks bounded | CONDITIONAL | Muxing into compressor tree must be checked during RTL synthesis. |
| Opcode cost controlled | PASS | Unused encodings explicitly reserved; no split-prefetch bit or 4-bit lane mode. |
| Precise exceptions defined | PASS | In-order retirement; faulting operations have no partial architectural update. |
| Memory ordering defined | PASS | Per-agent ordering plus explicit synchronization. |
| ABI and interrupt contract defined | PASS | Register classes, stack, stream ownership, and interrupt frame specified. |
| Toolchain contract defined | PASS | Assembler, compiler, debugger, and simulator requirements listed below. |
| Trace validation completed | PASS-ANALYTICAL | Hand-scheduled kernels and cycle models completed; RTL traces remain Phase 2. |
| Documentation complete | PASS | Overview, architecture spec, ISA reference, validation/freeze report, journal/checklist. |

# 3. Baseline implementation profile

32-bit register/data path; 16-bit instructions; eight-word instruction prefetch buffer.

Four-stage IF, ID/RR, EX/MEM, WB/RET pipeline.

In-order asymmetric dual issue with scalar Slot A and DSP/memory Slot B.

Two interleaved 32-bit closely coupled SRAM banks.

Two DMA channels.

One shared compressor/reduction engine and one iterative divider.

R0/R4 64-bit staging buffers and 128-bit coupled LIFO buffer.

Static backward-taken/forward-not-taken prediction; no dual-path prefetch.

# 4. Analytical kernel schedules

## 4.1 Four-tap packed FIR inner iteration

Assumption: R0 streams four signed 8-bit samples per access, R4 streams four signed 8-bit coefficients, and accumulator pair A0 is independent from the loop counter.

| Cycle | Slot A | Slot B | Comment |
| --- | --- | --- | --- |
| 0 | SUBS count, count, #1 | PMAC8 A0, R0, R4 | Consume both streams and accumulate four products |
| 1 | BNE loop | - | Branch issues alone; predicted taken |
| 2 | pointer/control work | PMAC8 next accumulator | Independent accumulator hides 2-cycle dependency where unrolled |
| Final | - | HADDW8 result, A0 | Reduce widened lanes |

Result: one four-lane MAC per computational issue cycle; loop-control cost is reduced by unrolling and scalar/DSP pairing. Zero-overhead loops remain a possible future extension only if measured branch overhead justifies it.

## 4.2 FFT butterfly primitive

| Step | Operations | Expected class |
| --- | --- | --- |
| 1 | PMUL16 twiddle products | 2-cycle compressor operation |
| 2 | PACK/UNPACK or ZIP lane arrangement | 1-cycle packed operation |
| 3 | PADD16 / PSUB16 butterfly outputs | 1-cycle packed operation |
| 4 | PRSHR16 optional fixed-point normalization | 1-cycle packed rounded shift |
| 5 | Stream stores | Scoreboarded memory operations |

No dedicated butterfly opcode is required for Phase 1. The kernel exercises packed multiply, lane rearrangement, add/subtract, rounded shift, and dual stream output.

## 4.3 Image block SAD

| Cycle | Slot A | Slot B |
| --- | --- | --- |
| 0 | loop/index update | SAD8 sum0, R0, R4 |
| 1 | address/control | SAD8 sum1, R0, R4 |
| 2 | ADD total, total, sum0 | stream/memory preparation |
| 3 | ADD total, total, sum1 | next SAD8 if independent destination |

SAD8 reuses packed subtract/absolute logic and the shared compressor tree. It avoids materializing four lane differences and a separate HADD instruction.

## 4.4 Matrix/PCA covariance dot product

| Cycle | Slot A | Slot B |
| --- | --- | --- |
| 0 | loop/index update | DOT16 pair0, R0, R4 |
| 1 | independent scalar work | DOT16 pair1, R0, R4 |
| 2 | ADD/ADC accumulator low/high | stream refill or next independent operation |
| Final | normalization preparation | HADD/shift as needed |

PCA and ICA receive acceleration from streams, widening dot products, MAC/MSUB, CLZ for normalization, rounded shifts, and matrix-friendly strides without dedicated algorithm-specific opcodes.

## 4.5 MCU control loop

| Cycle | Slot A | Slot B |
| --- | --- | --- |
| 0 | CMP sensor, threshold | LDW next_sample, [base] |
| 1 | conditional branch | - |
| 2 | ADD controller state | STW output, [peripheral] |

The same core supports ordinary control flow and strongly ordered peripheral access. DSP features do not replace the standard MCU execution model.

# 5. Toolchain and debug contract

Assembler must reject odd register-pair bases and unsupported lane/stream widths.

Assembler emits extension groups atomically and marks relocation records for long immediates, branches, streams, and DMA descriptors.

Linker preserves 2-byte instruction alignment and 8-byte ABI stack/data alignment where required.

Compiler backend models Slot A/Slot B pairing, scoreboard latency classes, stream ownership, caller/callee saves, and sticky Q.

Compiler correctness never depends on dual issue; every pair must remain valid as sequential instructions.

Debugger presents R0-R15, PSR, stream descriptors, DMA state, and paired 64-bit values.

Single-step treats extension prefix plus consumer as one architectural instruction group.

Simulator must model bank conflicts, stream underflow/write backpressure, exact faults, and configurable memory wait states.

Performance tools report issue-slot utilization, scoreboard stalls, bank conflicts, stream stalls, compressor occupancy, and branch penalties.

# 6. Phase 2 entry criteria

RTL skeleton implements decode, four-stage flow, scoreboard, SRAM-bank arbitration, and precise retirement.

Synthesis estimates quantify decoder, scoreboard, compressor muxing, SRAM interfaces, and critical paths.

Instruction-set simulator passes normative corner-case tests.

Assembler encodes every allocated opcode and rejects every reserved/illegal combination.

Kernel traces are reproduced cycle-accurately in simulation.

Architecture changes after this point require review-checklist approval and a versioned compatibility decision.