Aurora v1.2 RTL Architecture Specification

AUR-RTL-001 | Canonical Draft 1

# 1. Module Hierarchy

| Module | Responsibility |
| --- | --- |
| aurora_core_top | Top-level core integration |
| aurora_front_end | Fetch, prefetch, branch prediction, extension assembly |
| aurora_issue_window | Four-entry decode window, selection, compaction |
| aurora_regfile | 16 x 32 register file and writeback arbitration |
| aurora_lane0 / aurora_lane1 | Symmetric general execution lanes |
| aurora_lsu | Load/store unit and address generation |
| aurora_mul_reduce | Multiplier, compressor and reduction resources |
| aurora_divider | Multi-cycle divider |
| aurora_stream_q0/q1 | Stream state, staging, circular addressing |
| aurora_scoreboard | Busy state and completion tags |
| aurora_commit | In-order retirement and precise fault handling |
| aurora_interrupt | Priority arbitration and vector entry |
| aurora_memory_fabric | TCM/SRAM/MMIO/DMA/external arbitration |

# 2. Interface Rules

Ready/valid request-response interfaces between decoupled units

Explicit age tags for issued/completion records

No combinational path from external ready to fetch critical control

Symmetric lane dispatch; shared resources selected by resource masks

Pair destinations write both halves atomically

# 3. Clock and Reset

Single core clock baseline; optional peripheral/timer clock domains

Synchronous active-high internal reset preferred

All cross-domain status signals synchronized

Timer and DMA domains must not create variable interrupt semantics

# 4. Coding and Synthesis Rules

SystemVerilog synthesizable subset

No inferred latches

Parameterized SRAM/TCM interfaces

Assertions for protocol, ordering and pair-alias rules

Generated constants from the machine-readable ISA database